“ENGINEERING CHEMISTRY”ACADEMIC PLANNER

A/Y 2021-2022

I B.Tech SemesterI/ II

DEPARTMENT OF

HUMANITIES & SCIENCES

CMR ENGINEERING COLLEGE

(AUTONOMOUS )

(Affiliated to J.N.T.U. HYDERABAD)

KANDLAKOYA (V),MEDCHAL ROAD,

HYDERABAD – 501 401

ACADEMIC PLANNER (R-20)

LIST OF CONTENTS

1. Institute vision & mission

2. LIST OF POs

3. Mapping of course out comes with POs

4. Syllabus copy

5. Individual time table

6. Session plan

7. Session execution log

8. Assignment Questions and innovative assignments

9. Sample assignment script

10. Mid exam question papers

11. Scheme of evaluation

12. Sample mid answer script

13. Unit-wise course material

14. Material collected from Internet/Websites

15. Power point presentations

16. Innovation teaching methods (if any)

17. Previous question papers

18. References (Text books/websites/Journals)

HOD SIGNATURE

Institute mission and vision

Vision of the institute

To be recognized as a premier institution in offering value based and futuristic quality technical education to meet the technological needs

of the society.

Mission of the institute.

1.To impart value based quality technical education through innovative teaching

and learning methods.

2.To continuously produce employable technical graduates with advanced

technical skills to meet the current and future technological needs

of the society.

3.To prepare the graduates for higher learning with emphasis on academic and

industrial research.

LIST OF PEOs,POs AND PSOs

LIST OF POs:

Engineering Graduates will be able to:

1.Engineering knowledge: Apply the knowledge of mathematics, science, engineering fundamentals, and an engineering specialization to the solution of complex engineering problems.

2. Problem analysis: Identify, formulate, review research literature, and analyze complex engineering problems reaching substantiated conclusions using first principles of mathematics, natural sciences, and engineering sciences.

3. Design/development of solutions: Design solutions for complex engineering problems and design system components or processes that meet the specified needs with appropriate consideration for the public health and safety, and thecultural, societal, and environmental considerations.

4. Conduct investigations of complex problems: Use researchbasedknowledge and research methods including design of experiments, analysis and interpretation of data, and synthesis of the information to provide valid conclusions.

5. Modern tool usage: Create, select, and apply appropriate techniques, resources, and modern engineering and IT tools including prediction and modeling to complex engineering activities with an understanding of the limitations.

6. The engineer and society: Apply reasoning informed by the contextual knowledge to assess societal, health, safety, legal and cultural issues and the consequent responsibilities relevant to the professional engineering practice.

7. Environment and sustainability: Understand the impact of the professional engineering solutions in societal and environmental contexts, and demonstrate the knowledge of, and need for sustainable development

8. Ethics: Apply ethical principles and commit to professional ethics and responsibilities and norms of the engineering practice.

9. Individual and team work: Function effectively as an individual, and as a member or leader in diverse teams, and in multidisciplinary settings.

10. Communication: Communicate effectively on complex engineering activities with the engineering community and with society at large, such as, being able to comprehend and write effective reports and design documentation, make effectivepresentations, and give and receive clear instructions.

11. Project management and finance: Demonstrate knowledge and understanding of the engineering and management principles and apply these to one’s own work, as a member and leader in a team, to manage projects and in multidisciplinaryenvironments

12. Lifelonglearning: Recognize the need for, and have the preparation and ability to engage in independent and lifelonglearning in the broadest context of technological change.

LIST OF COs

COURSE OBJECTIVES

- To bring adaptability to the concepts of chemistry and to acquire the required skills to become a perfect engineer.

- To impart the basic knowledge of atomic, molecular and electronic modifications which makes the student to understand the technology based on them.

- To acquire the knowledge of electrochemistry and water treatment which are essential for the Engineers and in industry.

- To identify the type of corrosion and apply various principles for its preventions which are essential for an engineer in industry

- To impart the basic knowledge about solid fuel- coal, liquid fuels- petroleum, kerosene oil and diesel, gaseous fuels – LPG, natural gas their origin, classification, calorific value and its determination.

Course outcomes:

CO 1 :Identify different types of boiler troubles, choose appropriate method for softening and cleaning of water

CO 2: Define and differentiate atomic and molecular orbital, describe band theory for solid

CO 3: Compare and contrast the chemical behavior & physical properties of polymers .Identify the materials & methods that prevent   corrosion  in a particular environment

CO 4: Explain different types of electrode, electrode potential and determine EMF of a cell using Nernst equation

CO 5 : Understanding on different types of fuels, their synthesis and analysis

MAPPING OF COURSE OUT COMES WITH POS

| CO’s/ PO’s | PO1 | PO2 | PO3 | PO4 | PO5 | PO6 | PO7 | PO8 | PO9 | PO10 | PO11 | PO12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| C102.1 | 3 | 2 |  |  |  |  |  |  |  |  |  |  |
| C102.2 | 3 | 2 |  |  |  |  |  |  |  |  |  |  |
| C102.3 | 3 | 2 |  | 2 |  |  |  |  |  |  |  |  |
| C102.4 | 3 | 2 |  |  |  |  |  |  |  |  |  |  |
| C102.5 | 3 | 2 |  |  |  |  |  |  |  |  |  |  |
| AVG |  |  |  |  |  |  |  |  |  |  |  |  |

SYLLABUS COPY AND SUGGESTED/REFERENCE BOOKS

Unit –I	Water and its Treatment

- Introduction – hardness of water – causes of hardness – types of hardness: temporary and permanent – expression and units of hardness

- Estimation of hardness of water by complexometric method. Numerical problems.

- Boiler troubles: Sludge, scales and Caustic embrittlement.Boiler feed water and its treatment

- Internal treatment(Calgon conditioning, Phosphate conditioning and Colloidal conditioning).

- External Treatment- Softening of water by ion exchange processes.

- Potable water – specifications, steps involved in the treatment of potable water - Disinfection of potable water by chlorination and Ozonization Desalination of water -Reverse osmosis.

Unit-II	Molecular structure and Theories of Bonding:

Atomic and Molecular orbitals

- Linear Combination of Atomic Orbitals (LCAO),  molecular orbitals of diatomic molecules, molecular orbital energy level diagrams of - N2, O2, and F2 molecules.

- π-molecular orbitals of butadiene and benzene.

- Crystal Field Theory (CFT): Salient Features of CFT–Crystal Field Splitting of transition metal ion d-orbitals in Tetrahedral, Octahedral and square planar geometries.

- Band structure of solids and effect of doping on conductance.

Unit-III Electrochemistry & Batteries

- Electrochemistry: Electrochemical cells: Daniel cell – cell notation, cell reaction and cell EMF – Numerical problems. Electrochemical series and its applications. Nernst equation and its applications . Construction and functioning of Standard hydrogen electrode, calomel electrode, Quinhydrone and glass electrode. Determination of pH of a solution by using Quinhydrone and glass electrode.

- Batteries: Primary (Lithium cell) and secondary batteries (Lead – acid storage battery and Lithium ion battery).

- Fuel cells: Hydrogen–Oxygen fuel cell, Methanol-Oxygen fuel cell ,advantages and applications.

Unit –IV		      Fuels & Combustion

- Classification – solid fuels: coal – analysis of coal - proximate and ultimate analysis and their significance.

- Liquid fuels – petroleum and its refining – cracking – types – fixed bed catalytic cracking. Knocking – octane and cetane rating, synthetic petrol- Fischer-Tropsch’s process.

- Gaseous fuels - constituents, characteristics and applications of natural gas, LPG and CNG. Analysis of flue gas by Orsat’s apparatus – Numerical Problems.

- Combustion – Definition, Calorific value of fuel – HCV, LCV; Determination of calorific value by Junker’s gas calorimeter – theoretical calculation of Calorific value by Dulong’s formula – Numerical problems on combustion.

Unit-V

Engineering Materials and Corrosion

- Polymers: Introduction, classification and mechanism of polymerisation- Addition (Free radical polymerisation mechanism and Condensation polymerisation. Classification of polymers - Thermoplastics & Thermosetting resins. Types of Polymerization of polymers (i) Addition (ii) Condensation (iii) Co-Polymerization.

- Preparation, properties and engineering application of PVC, Teflon and Bakelite. Fibers- characteristics of fibers – preparation, properties and uses of Nylon – 6,6 and Dacron – Fiber Reinforced Plastics (FRP) Glass Fibres – applications. Rubber – Natural rubber and its vulcanization. Synthetic Elastomers- Buna S, Butyl rubber and Thiokol Rubber.

- Conducting polymers: Introduction, classification and mechanism of conduction in Poly-acetylene, applications of conducting polymers.

- Biodegradable polymers: Introduction preparation, properties and applications of polylactic acid

- Corrosion: Causes and effects of corrosion – theories of chemical and electrochemical corrosion – mechanism of electrochemical corrosion, Types of wet corrosion: Galvanic, water-line and pitting corrosion. Factors affecting rate of corrosion- nature of metal and nature of environment– Corrosion control methods – Cathodic protection (sacrificial anodic and impressed current). Surface coatings: Metallic coatings & methods of application of metallic coatings - hot dipping (galvanization & tinning), metal cladding, Electroless plating ( Ni plating).

SESSION PLAN/LESSON PLAN

| Unit 1 Water and its treatment |  |  |
| --- | --- | --- |
| Introductionhardness of water – causes of hardness | L1 | T2 |
| types of hardness: temporary and permanent – expression and units of hardness | L2 | T2 |
| Estimation of hardness of water by complexometric method | L3 | T2 |
| Numerical problems. | L4 | T2 |
| Boiler troubles: Sludges, scales and Caustic embrittlement.Boiler feed water and its treatment | L5 | T2 |
| Internal treatment(Calgon conditioning, Phosphate conditioning and Colloidal conditioning) | L6 | T2 |
| External Treatment- Softening of water by ion exchange processes | L7 | T2 |
| Potable water – specifications, steps involved in the treatment of potable water - Disinfection of potable water by chlorination and Ozonization Desalination of water – Reverse osmosis. | L8 | T2 |
| UNIT 2 Molecular structure and Theories of Bonding: Atomic and Molecular orbitals |  |  |
| Linear Combination of Atomic Orbitals (LCAO), molecular orbital of diatomic molecules | L9,L10 | T1,T4 |
| molecular energy level diagram of N2, O2, and F2 molecules | L11 | T1,T4 |
| π-molecular orbitals of butadiene and benzene | L12, | T1,T4 |
| Crystal Field Theory (CFT): Salient Features of CFT | L13 ,L14 | T1,T4 |
| Crystal Field Splitting of Transition metal ion d-orbitals in Tetrahedral, Octahedral and square planar geometries. | L15 ,L16 | T1,T4 |
| Band structure of solids and effect of doping on conductance | L17,L18 | T1,T4 |
| UNIT 3 Electrochemistry & Batteries |  |  |
| Electrochemistry: Electrochemical cells: | L19 | T2 |
| Daniel cell – cell notation, cell reaction and cell EMF | L20 | T2 |
| – Numerical problems.Electrochemical series and its applications- Nernst equation | L21 | T2 |
| Construction and functioning of Standard hydrogen electrode, calomel electrode,Quinhydrone and glass electrode. | L22,L23 | T2 |
| Determination of pH of a solution by using Quinhydrone and glass electrode | L24 | T2 |
| Batteries: Primary (Lithium cell) and secondary batteries (Lead – acid storage battery and Lithium ion battery). | L25,L26 | T2 |
| Fuel cells: Hydrogen–Oxygen fuel cell, Methanol-Oxygen fuel cell ,Advantages and Applications | L27 | T2 |
| UNIT 4 Fuels and Combustions |  |  |
| Classification – solid fuels: coal – analysis of coal - proximate and ultimate analysis and their significance | L28 | T1,T3 |
| Liquid fuels – petroleum and its refining – cracking – types – fixed bed catalytic cracking. | L29,L30 | T1,T3 |
| Knocking – octane and cetane rating, synthetic petrol, Bergius and Fischer-Tropsch’s process. | L31 | T1,T3 |
| Gaseous fuels - constituents, characterstics and applications of natural gas, LPG and CNG. Analysis of flue gas by Orsat’s apparatus – Numerical Problems. | L32,L33 | T1,T3 |
| Combustion – Definition, Calorific value of fuel – HCV, LCV | L34 | T1,T3 |
| Determination of calorific value by Junker’s gas calorimeter – theoretical calculation of Calorific value by Dulong’s formula | L35,L36 | T1,T3 |
| Numerical problems on combustion | L37 | T1,T3 |
| UNIT 5 Engineering Materials and Corrosion |  |  |
| Polymers: Introduction, classification and mechanism of polymerisation | L38 | T1,T4 |
| Addition (Free radical polymerisation mechanism and Condensation polymerisation | L38 | T1,T4 |
| Classification of polymers - Thermoplastics & Thermosetting resins. Types of Polymerization of polymers (i) Addition (ii) Condensation (iii) Co-Polymerization. | L39 | T1,T4 |
| Preparation, properties and engineering application of PVC, Teflon and Bakelite. Fibers | L40 | T1,T4 |
| characteristics of fibers – preparation, properties and uses of Nylon | L41 | T1,T4 |
| Dacron – Fiber Reinforced Plastics (FRP) Glass Fibres – applications | L41 | T1,T4 |
| Rubber – Natural rubber and itsvulcanization. Synthetic Elastomers- Buna S, Butyl rubber and Thiokol Rubber | L42 | T1,T4 |
| Conducting polymers: Introduction, classification and mechanism of conduction in Poly-acetylene, applications of conducting polymers | L43 | T1,T4 |
| Biodegradable polymers: Introduction preparation, properties and applications of polylactic acid | L44 | T1,T4 |
| Corrosion: Causes and effects of corrosion – theories of chemical and electrochemical corrosion – mechanism of electrochemical corrosion | L45 | T1,T4 |
| Types of wet corrosion: Galvanic, water-line and pitting corrosion | L46 | T1,T4 |
| Factors affecting rate of corrosion- nature of metal and nature of environment | L47 | T1,T4 |
| Corrosion control methods – Cathodic protection (sacrificial anodic and impressed current) | L48 | T1,T4 |
| Surface coatings: Metallic coatings & methods of application of metallic coatings - hot dipping (galvanization & tinning), metal cladding, Electroless plating ( Ni plating ) | L49 | T1,T4 |
| TOTAL NO OF CLASSES | 49 |  |

SESSION EXECUTION LOG PLAN

| UNIT | NAME OF THE TOPIC | DURATION |
| --- | --- | --- |
| 1 | Water and its Treatment: Introduction hardness of water Causes of hardness Types of hardness: temporary and permanent – expression and units of hardness Estimation of hardness of water by complexometric method. Potable water and its specifications Steps involved in treatment of water Disinfection of water by chlorination and ozonization Boiler feed water and its treatment – Calgon conditioning, Phosphate conditioning and Colloidal conditioning. External treatment of water Ion exchange process Desalination of water Reverse osmosis. Numerical problems |  |
| 2 | Unit - II: Unit - I: Molecular structure and Theories of Bonding Atomic and Molecular orbitals Linear Combination of Atomic Orbitals (LCAO) molecular orbitals of diatomic molecules, molecular orbital energy level diagrams of N2, O2 and F2 molecules π molecular orbitals of butadiene and benzene. Crystal Field Theory (CFT): Salient Features of CFT Crystal Field Splitting of transition metal ion d- orbitals in Tetrahedral, Octahedral and square planar geometries Band structure of solids and effect of doping on conductance |  |
| 3 | Unit - III: Electrochemistry and batteries Electro chemical cells – electrode potential, standard electrode potential Types of electrodes – calomel, Quinhydrone and glass electrode Nernst equation Determination of pH of a solution by using quinhydrone and glass electrode Electrochemical series and its applications. Numerical problems Batteries – Primary (Lithium cell) and secondary batteries (Lead – acid storage battery and Lithium ion battery) |  |
| 4 | Unit - IV: Fuels & Combustion Classification – solid fuels: coal – analysis of coal - proximate and ultimate analysis and their significance. Liquid fuels – petroleum and its refining – cracking – types – fixed bed catalytic cracking. Knocking – octane and cetane rating, synthetic petrol- Fischer-Tropsch’s process. Gaseous fuels - constituents, characteristics and applications of natural gas, LPG and CNG. Analysis of flue gas by Orsat’s apparatus – Numerical Problems. Combustion – Definition, Calorific value of fuel – HCV, LCV; Determination of calorific value by Junker’s gas calorimeter – theoretical calculation of Calorific value by Dulong’s formula – Numerical problems on combustion. |  |
| 5 | Unit - V:ENGINEERING MATERIALS AND CORROSION Polymers: Introduction, classification and mechanism of polymerisation- Addition (Free radical polymerisation mechanism and Condensation polymerisation. Classification of polymers - Thermoplastics & Thermosetting resins. Types of Polymerization of polymers (i) Addition (ii) Condensation (iii) Co-Polymerization.Preparation, properties and engineering application of PVC, Teflon and Bakelite. Fibers- characteristics of fibers – preparation, properties and uses of Nylon – 6,6 and Dacron – Fiber Reinforced Plastics (FRP) Glass Fibres – applications. Rubber – Natural rubber and its vulcanization. Synthetic Elastomers- Buna S, Butyl rubber and Thiokol Rubber. Conducting polymers: Introduction, classification and mechanism of conduction in Poly-acetylene, applications of conducting polymers. Biodegradable polymers: Introduction preparation, properties and applications of polylactic acid Causes and effects of corrosion – theories of chemical and electrochemical corrosion mechanism of electrochemical corrosion, Types of corrosion: Galvanic, water-line and pitting corrosion Factors affecting rate ofcorrosion, Corrosion control methods- Cathodic protection – Sacrificial anode and impressed current cathodic methods Surface coatings – metallic coatings – methods of application Electroless plating of Nickelcorrosion, Corrosion control methods- Cathodic protection – Sacrificial anode and impressed current cathodic methods Surface coatings – metallic coatings – methods of application Electroless plating of Nickel |  |

ASSIGNMENT QUESTIONS  AND INNOVATIVE ASSIGNMENTS

I BTECH-IISEM YEAR 2021-22 ASSIGNMENT-I

BRANCH: CSE,DS,IT,CS

SUBJECT: ENGINEERING CHEMISTRY Max. Marks : 05

---------------------------------------------------------------------------------------------------------

Q1. Discuss the various types Boiler troubles and Explain ion exchange method? (C0 1)

Q2. Describe the EDTA method for determining the hardness of water and explain Potable water treatment? (C0 1)

Q3. Explain salient futures of LCAO method and draw the molecular orbital diagram of N 2 and O 2 molecule .(C0 2)

Q4 Explain salient futures of CFT and explain the splitting of d orbital in Octahedral and tetrahedral complexes? (C0 2)

Q5. a.Describe the band structure of theory and discuss the effect of doping (C0 2)

b. Discuss and explain the working of Electrochemical cell? (C0 3)

I B.TECH II SEM II MID ASSIGNMENT

ENGINEERING CHEMISTRY

Date : 31-7-2021		MID-2 Assignment QUESTION PAPER

Q1.(a). How can you determine the pH of unknown solution by using quinhydrone electrode? [CO3]

(b) Explain the construction and working of Pb-Acid Battery?

(C ) Derive Nerns’t equation for electrode potential?

Q2. (a).How pH of a solution is determined by Glass electrode? Discuss? [CO3]

(b) Describe Methanol Oxygen fuel Cell ?[CO3]

Q3. (a). Explain analysis of flue gases by Orsat Apparatus?  [CO4]

(b) Explain Refining of Petroleum?

Q4.(a). Explain Classification of Coal?.  [CO4]

(b) Explain Ultimate Analysis of Coal?

(c) How to determine calorific value of gaseous fuel by Junker’s colorimeter?

Q5.(a). Explain the reaction involved in Electro chemical corrosion?

(b)Explain galvanizing and metal cladding?  [CO3]

(c) What is Vulcanisation of Natural Rubber?

(d) Write the preparation properties of Bakelite, PTFE, Polyester?

(e) Write the preparation properties of BUNA-S, BUNA-N,BUTYL RUBBER, POLYSULPHIDE RUBBER(THIOKOL RUBBER) ?

INNOVATIVE ASSIGNMENT/PROJECT BASED LEARN

(ENGINEERING CHEMISTRY)

2021-2022

Problem Statement

Identify different types of boiler troubles, methods for softening and cleaning of water .define and differentiation of atomic and molecular orbitals with decription of band theory of solids.  explain the different types of electrodes, electrode potential with derivation of nernst equation. synthesis and analysis of different types of fuels. comparison of various properities of polymers with classification and preventive methods of corrosion .

Rubrics

| No | Parameter | GRADE 3 | GRADE 2 | GRADE 1 |
| --- | --- | --- | --- | --- |
| I | Identify different types of boiler troubles, choose appropriate method for softening and cleaning of water CO : 1 | Statement of question is presented and necessary procedure is explained. | Statement of question is presented but necessary procedure is not explained properly . | Statement of question is not presented and necessary procedure is not explained. |
| II | Define and differentiate atomic and molecular orbital, describe band theory for solid CO: 2 | Differentiation and description is explained | Differentiation is explained but description is not explained. | Only definition is given but differentiation and description is not explained. |
| III | Explain different types of electrodes, electrode potential and determine EMF of a cell using Nernst equation CO :3 | Applications of Electrodes, EMF and Nernst equation are clearly explained with suitable examples and proper derivation is done | Applications of of Electrodes, EMF and Nernst equation are clearly explained with suitable examples but no proper derivation is done | Only few applications are stated along with no proper derivation |
| IV | Understanding on different types of fuels, their synthesis and analysis CO:4 | Analysis and synthesis of different types of fuels is efficiently done with proper calculations | Analysis and synthesis of different types of fuels is efficiently done but without proper calculations. | Synthesis of all fuels is done but without proper analysis and calculations |
| V | Compare and contrast the chemical behavior & physical properties of polymers .classify the different types of corrosion with proper proper preventive methods of corrosion CO : 5 | Understanding the various properties of polymers and identify the problems with proper preventive methods | Understanding the various properties of polymers without identifying the problems | Only few proprieties are stated without identifying the problems |

Calculation of Attainment Values

| Roll No | Parameter I | Parameter II | Parameter III | Parameter IV |  |
| --- | --- | --- | --- | --- | --- |
| 20 - 401 |  |  |  |  |  |
| 20 – 402 |  |  |  |  |  |
| 20 – 403 |  |  |  |  |  |
| …. |  |  |  |  |  |
| …. |  |  |  |  |  |
| Attainment level | 1,2,3 | 1,2,3 | 1,2,3 | 1,2,3 |  |

| Course Outcome | Parameters | (Sample) Average Attainment Value |
| --- | --- | --- |
| CO 1 | I |  |
| CO 2 | II |  |
| CO 3 | III |  |
| CO 4 | IV |  |
| CO 5 | V |  |

| Program Outcome | Parameters | Average Attainment Value |
| --- | --- | --- |
| PO 1 | CO 1, 2,3,4,5 | Avg(CO1 + CO2+CO3+CO4 +CO5) |
| PO 2 | CO1,2,3,4,5 | Avg(CO1+CO2+CO3+CO4+CO5) |
| PO 3 |  |  |
| PO 4 | CO 3 | CO 3 |

MID EXAM PAPERS ALONG WITH  SAMPLE ANSWER SCRIPTS

I.B.TECH- II-SEM                        MID-I  EXAMINATION                       Date:  Time: 09.30AM to 11:00AM

SUBJECT: ENGINEERING CHEMISTRY		                  Branch: H&S                          Marks: 25 M

Note: Question paper contains two parts,Part - A and Part - B.

Part-A is compulsory which carries 10 marks. Answer all questions in part-A.

Part-B consists of (21/2) units. Answer any one full question from each unit. Each question carries 5marks and may have a,b,c sub questions.

PART-A                                                  5x2=10

Q1. Draw the neat diagram of Ion exchange method?	                                              [CO1]

Q2. Write a brief account of Break point Chlorination?	                                              [CO1]

Q3. Discuss the rules of LCAO method?                                                                          [CO2]

Q4. Explain is the various steps of removal of suspended impurities ?		       [CO1]

Q5. Writethe salient features of CFT ?	                                                                         [CO2]

PART-B                                                             3X5=15

- Describe the principle of EDTA method for determining the hardness of water?[CO1]

(or)

- What is the principle involved in treatment of boiler feed water and discuss the types of internal treatment methods?.                                                                                                                                           (CO1)

- Explain the splitting of d orbital  in Octahedral and tetrahedral complexes?(CO2)

(or)

- Explain the Molecular orbital  diagram of O2and  N2.                                                                                                  (CO2)

- Discuss the band structure of Solids and doping?                                                                       (CO2)

(or)

- Discussand explain the working of Electrochemical cell?.                                                      (CO3)

SCHEME OF EVALUATION

DESCRIPTIVE TEST - I                  A.Y:  2020-21

Branch    :  Common to ALL 						 Subject : ENGG.CHEM

Course    :  I B.Tech  (SEM  I & II)     		 Max marks:25

SCHEME OF EVALUATION

| S.No |  | THEORY | MARKS | TOTAL |
| --- | --- | --- | --- | --- |
| PART -A | A | DEFINITION | 1 | 2 |
|  |  | FORMULA | 1 |  |
|  | B | DEFINITION | 1 | 2 |
|  |  | FORMULA | 1 |  |
|  | C | DEFINITION | 1 | 2 |
|  |  | FORMULA | 1 |  |
|  | D | DEFINITION | 1 | 2 |
|  |  | FORMULA | 1 |  |
|  | E | DEFINITION | 1 | 2 |
|  |  | FORMULA | 1 |  |
| PART –B | Q NO.2 | DEFINITION | 1 | 5 |
|  |  | DIAGRAM | 1 |  |
|  |  | FORMULAS | 1 |  |
|  |  | DESCRIPTION | 1 |  |
|  |  | APPLICATIONS | 1 |  |
|  | OR |  |  |  |
|  | Q NO.3 | DEFINITION | 1 | 5 |
|  |  | DIAGRAM | 1 |  |
|  |  | FORMULAS | 1 |  |
|  |  | DESCRIPTION | 1 |  |
|  |  | APPLICATIONS | 1 |  |
|  | Q NO.4 | DEFINITION | 1 | 5 |
|  |  | DIAGRAM | 1 |  |
|  |  | FORMULAS | 1 |  |
|  |  | DESCRIPTION | 1 |  |
|  |  | APPLICATIONS | 1 |  |
|  | OR |  |  |  |
|  | Q NO. 5 | DEFINITION | 1 | 5 |
|  |  | DIAGRAM | 1 |  |
|  |  | FORMULAS | 1 |  |
|  |  | DESCRIPTION | 1 |  |
|  |  | APPLICATIONS | 1 |  |
|  | Q NO.6 | DEFINITION | 1 | 5 |
|  |  | DIAGRAM | 1 |  |
|  |  | FORMULAS | 1 |  |
|  |  | DESCRIPTION | 1 |  |
|  |  | APPLICATIONS | 1 |  |
|  | OR |  |  |  |
|  | Q NO. 7 | DEFINITION | 1 | 5 |
|  |  | DIAGRAM | 1 |  |
|  |  | FORMULAS | 1 |  |
|  |  | DESCRIPTION | 1 |  |
|  |  | APPLICATIONS | 1 |  |

SAMPLE MID ANSWER SCRIPTS

UNIT WISE COURSE MATERIAL

UNIT-I

WATER AND ITS TREATMENT

The water which does not produce lather or produce very little lather with soap is called hard water. This is due to presence bicarbonates, sulphates, chlorides and nitrates salts of some metal ions like Ca+2, Mg+2 and other Heavy metal. Soaps are Sodium or Potassium salts of higher fatty acids like Oleic acid or Palmitic acid or stearic acids. A sample of hard water, when treated with soap doesn’t produce lather but forms white precipitate. This ppt is formed due to formation of insoluble salts of Ca and Mg.

2C17H35COONa + CaCl2 (C17H35COO)2 Ca + 2NaCl

TYPES OF HARDNESS

Depending on the salts presents in water hardness is divided into two types.

- Temporary hardness (carbonate hardness).

- Permanent hardness (non Carbonate Hardness

- Temporary Hardness:- This is due to the presence of dissolved Bicarbonates of Calcium, Magnesium salts.

- Temporary hardness responsible salts are

Calcium bicarbonate  Ca(HCO3)2,

Magnesium bicarbonate  Mg(HCO3)2.

It is removed by boiling of water, when bicarbonates are decomposed, yielding insoluble carbonates or hydroxides

2. Permanent hardness : -

It is due to the presence of chlorides and sulphates of calcium, magnesium ions and other heavy metals. These salts cannot be removed by boiling. They are removed by different methods.
	Permanent hardness responsible salts are…. CaCl2, MgCl2, CaSO4, MgSO4.

These salts cannot be removed by boiling.

CAUSES OF HARDNESS:

- In Domestic use:

WASHING: Hard water, when used for washing purposes, does not producing lather freely with soap. As a result cleaning quality of soap is decreased and a lot of soap is wasted. Hard water reacts with soap it produces sticky precipitates of calcium and magnesium soaps. These are insoluble formations. Continuous laundering in hard water can damage fibers and shorten the life of clothes by up to 40 percent.

Bathing: Hard water does not lather freely with soap solution, but produces sticky scum on the bath-tub and body. Thus, the cleaning quality of soap is depressed and may lead to irritation. Soap curd on hair may make it dull, lifeless and difficult to manage.

Cooking: The boiling point of water is increased because of presence of salts. Hence more fuel and time are required for cooking.

Drinking: Hard water causes bad effects on our digestive system. Moreover, the possibility of forming calcium oxalate crystals in urinary tracks is increased.

- INDUSTRIAL USE:

Textile Industry: Hard water causes wastage of soap. Precipitates of calcium and magnesium soaps adhere to the fabrics and cause problem.

Sugar Industry: Water containing sulphates, nitrates, alkali carbonates. Etc. if used in sugar refining, causes difficulties in the crystallization of sugar.

Dyeing Industry: The dissolved salts in hard water may reacts with costly dyes forming precipitates.

Paper Industry: Calcium, magnesium, Iron salts in water may affect the quality of paper.

Pharmaceutical Industry: Hard water may cause some undesirable products while preparation of pharmaceutical products.

Concrete Making: Water containing chlorides and sulphates, if used for concrete making, affects the hydration of cement and the final strength of the hardened concrete.

Laundry: Hard water, if used in laundry, causes much of the soap used in washing to go as waste Iron salts may even cause coloration of the clothes.

HARDNESS OF WATER.

We know that hardness of water is due to the presence of dissolved salts in water, but comparing the hardness of different samples of water of varying composition, it is necessary to choose a reference standard. For this purpose, hardness of water is expressed in terms f equivalents of CaCO3. Hence whatever amount of dissolved salt is present in water; it is first converted into calcium carbonate equivalents by using the following formula:

Equivalents of CaCO3 = weight of dissolved salt X 100/GMW of dissolved salt.

UNITS OF HARDNESS.

- Parts per million (ppm) Is the parts of calcium carbonate or its equivalents of other salts present in 106 parts of water, i.e.,

1ppm  =1 part of CaCO3 eq hardness present in 106 parts of water.

- Milligrams per litre (mg/L) Is the number of milligrams of CaCO3  or its equivalents of other salts present in one litre of water.

Thus: 1 mg/L = 1 mg CaCO3 eq. hardness in 1 Lof water.

But 1 L of water weight = 1 kg = 1000 gm = 1000x1000 mg = 106 mg.

.∙. 1 mg/L = 1 mg of CaCO3 eq per 106 parts of water = 1ppm.

- Clarke’s degree (0Cl). Is the number of grains of CaCO3 or its equivalents of other salts present in one gallon of water. Or it is parts of CaCO3 equivalent hardness per 7000 parts of water. Thus :

1 OClarke = 1 grain of CaCO3 eq hardness per gallon of water. Or

1 OClarke = 1 part of CaCO3 eq hardness per 70,000 parts of water.

- Degree French (0Fr) Is the parts of CaCO3 or its equivalents of other salts present in 105 parts of water. Thus: 1 0Fr = q part of CaCO3 hardness eq per 105 parts of water.

- Milliequivalent per litre ( meq/L) is the number of milli equivalents of CaCO3or its equivalents of other salts present per litre. Thus:

1 meq/L = 1 meq of CaCO3 per L of water

Relation among various units of hardness.

1 ppm 		= 1 mg/L 	= 0.10Fr 	= 0.07 0Cl 	= 0.02 meq/L

ESTIMATION OF TEMPORARY AND PERMANENT HARDNESS OF WATER:

EDTA METHOD:

It is the most important and more accurate method to determine the hardness of water. EDTA has limited solubility in water, he nce disodium salt of EDTA is used which is soluble in water.

PRINCIPLE: In this complex metric Titration, the water sample is titrated with standard solution of Di sodium salt of EDTA using EBT indicator. EDTA can form complex with Ca+2 and Mg+2 salts which are present in hard water. Hence it is called complexometric titration.with

To the hard water sample Erichrome Black T indicator is added along with ammonia buffer solution. EBT forms an unstable, whine red color complex Ca+2 and Mg+2 ions.

(Ca2+ / Mg2+) + EBT    [Ca/Mg– EBT] complex

Blue           wine-red

During titration with EDTA, calcium first reacts to form relatively stable followed by magnesium to give Mg-EDTA complex releasing the free EBT indicator. The color changes from wine-red to blue at the endpoint.

[Ca/Mg– EBT] complex + EDTA  [Ca/Mg– EDTA] complex + EBT

Wine-red                                               colorless               blue

Preparation of Standard hard water:-1 g  of pure CaCO3 is dissolved in minimum quantity of HCl and then made up to 1000ml using distilled water.

Molarity of standard CaCO3 is 0.01M

Standardization of EDTA solution:

Fill burette with EDTA solution up to the mark. Pipette out 20 ml of given standard hard water solution into a 250ml conical flask. To this add 5ml of buffer solution and add 2-3 drops of EBT indicator. Then it changes to wine red color and it is titrated against EDTA solution from burette. At the end point the color changes from wine red to blue. Note down the volume of EDTA run down for titration. Repeat the same procedure to get concurrent values

Let the volume of EDTA consumed be ‘x’ ml.

Determination of total hardness of water:

Pipette out 20ml of sample water (bore well water or any other source of hard water) into conical flask, to this add 5ml of buffer solution, 2-3 drops of EBT indicator the solution becomes wine red in color. Titrate this solution against EDTA solution from burette until wine red color changes to blue. This is the end point. Note down the volume of EDTA run down for titration.

Let the volume of EDTA consumed be ‘y’ ml.

Estimation of permanent hardness

250 ml of hard water sample is taken in a beaker and evaporated to nearly 50 ml. filter and wash and make up the filtrate to 250 ml with distilled water. 20 ml of made up solution is pipette out into a conical flask. , to this add 5ml of buffer solution, 2-3 drops of EBT indicator the solution becomes wine red in color. Titrate this solution against EDTA solution from burette until wine red color changes to blue. This is the end point. Note down the volume of EDTA run down for titration. Repeat the titration to get concurrent values.

Let the volume of EDTA consumed be ‘z’ ml.

Calculations:-

- Standardization of EDTA

V1M1 = V2 M2

Volume of standard hard water = V1 = 20ml

Molarity of standard hard water = M1 = 0.01 M

Volume of EDTA consumed = V2 ml =  x ml

Molarity fo EDTA solution = M2 = ?

According to volumetric law

Molarity of EDTA (M2) =V1 M1/V2

= 20 X 0.01/x ml

- Estimation of total hardness of water sample

M2V2 = M3V3

Volume of  hard water = V3 = 20ml

Molarity of hard water = M3 = ?

Volume of EDTA consumed = V2 ml = y ml

Molarity fo EDTA solution = M2

According to volumetric law

M3 = M2V2/V3

= y ml x M2/20

Total hardness = (y ml X M2/20)100X1000 ppm

- Estimation of permanent hardness of water sample

V2M2 = V4M4

Volume of  hard water = V4 = 20ml

Molarity of hard water = M4 =?

Volume of EDTA consumed = V2 ml = z ml

Molarity fo EDTA solution = M2

According to volumetric law

M4 == V2 M2/V4

= z ml X M2/20

Permanent hardness = (z ml X M2 /20) 100 X 1000 ppm

- Estimation of temporary hardness

Temporary hardness = total hardness – permanent hardness

Advantages of EDTA method:

This method is definitely prep arable to the other methods, because of the

- Greater accuracy 2. Convenience and 3. More rapid procedure.

BOILER TROUBLES

Boiler troubles are mainly caused by the impurities present in the boiler feed water. The total removal of salts in boiler feed water is not made use of the scales produced in the boilers can act as insulators of heat. As a result of it more heat has to be supplied from outside of the boilers. The major boiler troubles are

- Scales and sludge formation

- Caustic embrittlement

(1) Scales and sludge formation

When water is evaporated in boilers to produce steam continuously the concentration of the salts present in the water increases progressively. As the concentration reaches a saturation point the salts are thrown out of water as precipitates either as sludges or as scales adhering to the walls of the boiler.

If the precipitate formed is soft, loose and slimy, it is called sludge. If the precipitate produced is hard and adheres very strongly to the walls of the boilers, it is known as scale.

Reasons for  scale and sludge formation.

- The solubility product of the salt must be exceeded by the product of concentration of constituent ions.

- The solubility of the salt decreases with rise of temperature

- The increase in the temperature can lead to reactions those results in the formation of insoluble.

- Scales:

Scales are hard deposits, which stick very firmly to the inner surface of the boiler. Scales are difficult to remove even with the help of hammer and chisel. Scales are the main source of boiler troubles.Scales may be formed inside the boiler due to:

- Decomposition of calcium bicarbonate

Ca (HCO3)2		CaCO3 ↓ + H2O + CO2

Scale

However scale composed chiefly of calcium carbonate is soft and is the main cause of scale formation in low – pressure boilers but in high pressure boilers, CaCO3 is soluble

CaCO3 + H2O		       Ca (OH) 2 + CO2

Soluble

(ii) Deposition of calcium sulphate :

The solubility of CaSO4 in water decreases with rise of temperature. Thus, solubility of CaSO4 is 3200ppm at 150C and it reduces to 55ppm at 300C and 27 ppm at 320C.  In other words, CaSO4 is soluble in cold water, but almost completly insoluble in super heated water. Consequently, CaSO4 gets precipitated as hard scale on the boiler. This is the main cause of scales in high – pressure boilers.

(iii) Hydrolysis of Magnesium salts.

Dissolved Mg salts undergo hydrolysis forming magnesium hydroxide precipitate, which forms a soft type of scale.

MgCl2 + 2H2O 	Mg (OH)2  ↓ + 2 HCl

Scale

(iv) Presence of Silica (SiO2)

Even present in small quantities, deposits as calcium silicate (CaSiO3) and / or Magnesium silicate (MgSiO3), these deposits stick very firmly on the inner side of the boiler surface and are very difficulty to remove.  One important source of silica in water is the sand filter.

Disadvantage of scale formation

(i) Wastage of Fuel: Scales have a poor thermal conductivity so the rate of heat transfer from boiler to inside water is greatly reduced. Inorder to provide a steady supply of heat to water excessive or over – heating is done and this causes increase in fuel consumption.

(ii) Lowering of boiler safety: Due to scale formation over heating of boilers is done in order to maintain a steady supply of steam. It makes the boiler material softer and weaker. This causes distortion of boiler tube and also makes the boiler unsafe to bear the pressure of the steam especially in high pressure boilers.

(iii) Decrease in efficiency: Deposition of scale in the valves and condensers of the boiler, choke them partially. This results in decrease in efficiency of the boiler.

(iv) Danger of explosion: When thick scales cracks due to uneven expansion, the water comes suddenly in contact with overheated portion and large amount of steam is formed instantaneously. This results in development of sudden high pressure which may cause explosion of the boiler.

Removal of Scales:

(i) Scales can be removed by applying thermal shocks. (Sudden heating and cooling)

(ii) Using scrapers, wire brush etc, scales can be removed.

(iii) Scales are removed by mechanical methods, i.e. if the scales are loosely adhering; it can be removed with the help of scrapers (or) piece of wood or wire brush

(iv) If the scales are loosely adhering they can also be removed by frequent blow – down operation. Blow –down operation is partial removal of hard water through a top of the bottom of the boiler, when extent of hardness in the boiler becomes alarmingly high.

Scales can also removed by chemical methods. If the scales are adherent and hard, they can be removed by dissolving them by adding chemicals.

Eg: CaCO3 scales can be dissolved by using 5-10% HCl. CaSO4 scales can be removed by adding EDTA, since the Ca – EDTA complex is highly soluble in water.

Prevention of scale formation by internal treatment:

SLUDGE: Sludge is a soft, loose and slimy deposit formed within the boiler. Sludge can easily scrap off with a wire brush. It is formed at comparatively colder portions of the boiler and collects in areas of the system, where the flow rate is slow or at bends. Sludges are formed by substances which have greater solubility in hot water than in cold H2O.

Eg : MgCO3, MgCl2, CaCl2, MgSO4 etc.

Disadvantages of sludge formation

- Sludges are poor conductor of heat, so they tend to waste a portion of heat generated.

- If sludges are formed along with scales, then former gets entrapped in the lather and both get deposited as scales.

- Excessive sludge formation disturbs the working of the boiler. It settles in the regions of poor water circulation, such as pipe connection, plug opening, there by causing even choking of the pipes.

Prevention of sludge formation

- By using well softened water.       (2)  By frequently blow – down operation.

*

Sludges and Scales in boiler*

CAUSTIC EMBRITTLEMENT

Caustic embrittlement is a type of boiler corrosion, caused by using highly alkaline water in the boiler. During softening process by lime – soda process, free Na2CO3 is usually present in small amounts in the softened water. In high pressure boilers, Na2CO3 decomposes to give NaOH and CO2 and this makes the boiler water “Caustic”.

Na2CO3 + H2O		2NaOH + CO2 (g)

The NaOH containing water flows into the minute hair – cracks always present in the inner side of boiler, by capillary action. Here water evaporates and the dissolved caustic soda concentration increases progressively. This caustic soda attacks the surrounding area, thereby dissolving iron of boiler as sodium ferrite (Na2FeO2). This causes embrittlement of boiler parts, particularly stressed parts (like bends, Joints, rivets, etc), caustic even failure of the boiler. Caustic cracking can be explained by considering the following concentration cell:

| (-)Anode Iron at rivets, bends, joints, etc | Con Concentrated NaOH solution | dilute NaOH Solution | cathode(+) Iron at Pla ne surfaces |
| --- | --- | --- | --- |

The iron surrounded by the dilute NaOH becomes the cathodic side, while the iron in contact with concentrated NaOH – become anodic part, which is consequently dissolved or corroded.

Prevention of Caustic embrittlement:

- By using sodium phosphate (Na3PO4) as softening reagent, instead of sodium carbonate (Na2CO3)

- Neutralising the alkali (NaOH) with a very small amount of acid

- By adding tannins, or lignin to boiler water, since these blocks the hair cracks, thereby preventing infiltration of caustic soda solution in these.

By adding small amount of Na2SO4 to boiler water in order to block small hair cracks at stressed areas.

Potable water:

Potable water is water which is fit for consumption by humans and other animals. It is also called drinking water

Ex: municipal water, purified by reverse osmosis etc

The following are the specifications of water drinking purpose.

POTABLE WATER TREATMENT

The treatment of water for drinking purposes mainly includes the removal of suspended impurities, colloidal impurities and harmful pathogenic bacteria. The following is the flow diagram of the water treatment for domestic purposes.

Sterilization and disinfestation

Storage and distribution

Filtration

Sedimentation

Aeration

Screening

Source of water

Flow diagram of treatment of water for municipal supply

- Screening: - It is used to remove suspended impurities present in water. In this process the raw water is passed through screens having large number of holes, where floating impurities like rags, papers, wood pieces, and leaves etc., are held by them.

- Sedimentation:

It is the process of removing tine, suspended, colloidal impurities by allowing water for certain period in big tanks about 5 m deep or to flow quietly at low velocities. Most of the suspended particles settle down at the bottom, due to the force of gravity. This process takes 2 to 8 hours. The clear water is then drawn from the tank with the help of pumps.

Fine suspended particles like mud and colloidal particles present in the water cannot settle by plain sedimentation. They are removed by sedimentation with coagulation.

Sedimentation with coagulation:

It isthe process of removing fine suspended and colloidal impurities by addition of requisite amount of chemicals (coagulants) to water  The coagulants mainly used are alum, ferrous sulphate etc, they provide positive ions like Fe+3, Al+3 which neutralize the negative charge on the colloidal clay particles. After losing their charge the tiny particles come nearer to one another and combine to form bigger particles, which settle down, due to the force of gravity.

- Filtration:

It is the process of separating colloidal and suspended impurities from water by passing it through a porous bed made of fine sand and other proper mixed granular materials. The filters  consists of a top thick layer of fine sand, placed over coarse sand layer followed by gravel. Water comes from the top. The suspended particles present in water that are trained there itself and the water becomes free of them. The sand layer may get choked after sometime and then it is to be replaced with clean sand for further action.

Sterilization and disinfestation:

It is the process of removing pathogenic bacteria, viruses and protozoa from water. A large number of methods are utilized, some of them are as follows.

- Boiling:  it is the simplest and easiest method to disinfect water. But it is useful only for small quantities of water. For proper disinfection, water should be boiled vigorously for 20 – 25 minutes.

- Chlorination: The process of utilizing chlorine as a powerful disinfectant is called chlorination. There are three types of chlorinating reagents.

i)	By passing chloramines: Chlorine is mixed with ammonia in the ratio 2:1 by volume to form a stable chloramine which generates hypochlorous acid, a powerful disinfectant

Cl2+NH3	------------->	ClNH2+ HCl

ClNH2 + H2O	------------>	HOCl +NH3

Hypochlorousacid (HOCL) inactivates the enzymes of bacteria and kills bacteria. Chloramines are useful for disinfecting of swimming pools.

Advantages:

- It gives good taste to treated water.

- Excess chloramine does not produce irritating odor.

ii)	By bleaching powder : Bleaching powder contains 80%chlorine. About 1 kg of bleaching powder per 1000 kiloliters of water is mixed and water allowed to stand undisturbed for several hours. The chemical action produces hypochlorous acid.

CaOCl2 + H2O ------------->	Ca(OH)2 + Cl2

Cl2 + H2O ------------->	HOCl +HCl

(Disinfectant)

Drawabacks :

- Bleaching powder introduces calcium in water, thereby making it more hardness.

- Only calculated quantity of bleaching powder should be used, since an excess of it gives a bad taste and smell to treated water.

- Chlorination: The process of applying calculated amount of chlorine to water in order to kill the pathogenic bacteria is called chlorination. Chlorine also reacts with water and generates hypochlorous acid, which kills bacteria. Chlorine is a powerful

Cl2 +H2O 	------------->	HOCl +HCl

(Hypochlorous acid)

Chlorine is most widely used disinfectant throughout the world. Apparatus used for this purpose is known as chlorinator, which is a high tower, having a number of baffle plates. Water and proper quantity of concentrated chlorine solution are introduced at its top. During their passage through the tower, they get thoroughly mixed. The treated water is taken out from the bottom.

Break point chlorination:

The point at which the added chlorine completely removes bacteria, NH3, Organic and inorganic impurities is known as “Breakpoint chlorination”.

Water contains: a) Bacteria b) Ammonia c) O rganic impurity (sewage)     d) Inorganic salt impurities (Effluents, H2S, Fe salts).

1. When we add chlorine, first it oxidizes some inorganic reducing impurities. The added chlorine is completely consumed. (a).

2. Further addition of chlorine is used to combine with ammonia to form chloramines compounds.  So the residual chlorine content is increased up to (b).

3. As saturation level is attained, the chloramines begin to decompose to release chlorine which is utilized to remove all the types of organic and inorganic impurities. And kill the bacteria, so residual chlorine level decreases. (c).

4. At point (c) the added chlorine removed all the types of impurities.  So the point is known as “Break point chlorination”.

6. After this point, further addition of chlorine is not at all utilized and simply increases the residual chlorine content. (along c d)

Diagram for break point chlorination:

Advantages of break point chlorination

- It removes taste, colour, and oxidizes completely organic compounds, ammonia and other reducing impurities.

- It destroys completely all disease producing bacteria.

- It introduces no salt impunities in treated water.

- It is effective and economical.

Drawbacks :

- Excess of chlorine, if added, produces a characteristic unpleasant test and odor.

- Excess chlorine produces an irritation on mucous membrane.

- It is more effective below 6.5 PH.

Ozonisation :

Ozone when passed into water acts as disinfectant. Ozone is unstable isotope of oxygen, produces nascent oxygen which is a powerful disinfectant. This treatment is costly and ozone is unstable and cannot be stored for a longtime.

O3 → O2 + [O]

(Nascent Oxygen)

The nascent oxygen is very powerful oxidizing agent and kills all bacteria as well as oxidizes the organic matter present in water.

For carrying out the disinfection by ozone,ozone is released/injected  in to the water and the tqwo are allowed to come in contact in a sterilizing tank.the disinfected water is removed from the top.the contact period is 10-15 minutes. And the usual dose strength is 2-3 ppm.

UNIT -II

MOLECULAR STRUCTURE AND THEORIES OF BONDING: ATOMIC AND MOLECULAR ORBITALS

Introduction

Orbit: Electrons are revolving around the nucleus in fixed circular path is called orbit

1st shell = 2 electrons

2nd shell = 8 electrons

3rd shell = 18 electrons

Nucleus of atom

k = 2 e-		L = 8 e-	        M = 18 e-                 N = 32 e-

Orbitorbital

K    ---      ls	       L    ------   2s 2p	                   M------   3s 3p 3d	              N   -------   4s 4p4d 4f

ORBITAl: The three dimensional region around the nucleus where the probability of finding the electrons is maximum is called orbital. There are mainly four types of orbitals i.e.  s, p, d, f

S-orbital–shape is spherical and P-orbital–shape is dumb-bell, the three possible p-orbital’s are always perpendicular to each other. D-orbital–shape is double dumb bell.

S = sharp

P = principal

D = diffuse

F = fundamental

Structural representation of Orbitals

Atomic orbitals:	Atomic orbital is the region having the highest probability of finding an electron in an atom.

- The shape is determined by the type of atomic orbital (s, p, d &f) overlapping of atomic orbitals.

When orbitals of two atoms come close to form bond:

- Sigma () bond:

This is formed by the end to end (head–on) overlap of bonding orbitals alone the inter-nuclear axis.

- S–S overlapping

In this, there is overlap of two half filled s–orbitals along the inter-nuclear axis.

+

S – orbital      s – orbital  		s – s overlapping

- S–P overlapping

This type of overlap occurs between half filled s–orbitals of one atom and half filled–orbitals of another atom.

- P– P overlapping

This type of overlap takes place between half filled p–orbitals of the two approaching atoms.

2)   Pi (π) bond:

In the formation of π–bond, the atomic orbitals overlap in such a way that their axis remains parallel to each other & perpendicular to inter nuclear axis. This type of overlapping is called lateral overlap.

Molecular orbitals

- Molecular orbital is a function describing the wave like behavior of an electron in a molecule.

- By molecular orbital, we know the probability of finding an electron in any specific region.

- When two atomic orbitals combine, two molecular orbitals are formed, one is known as bonding molecular orbital. While the other is called anti bonding molecular orbital.

- The bonding MO’s are lower energy than ABMO’s

| Atomic | Molecular orbital |
| --- | --- |
| - AO is the region having the highest probability of finding an electron in on atom - Formed by e- cloud around the atom - The shape is determined by the type of AO (s,p,d and f) - Schrodinger equation is used - Single nucleus affects the electron cloud | - MO’ is the region having the highest probability of finding an electron in a molecule. - Formed by fusion of AO’s that have nearly the same energy. - The shape is determined by the shapes of AO’s that makes the molecule. - LCAO is used - Two or more nuclei affect the electron cloud. |

Linear combination of Atomic orbitals  (LCAO)

- According to Schrödinger wave equation an atomic orbital can be represented in terms of its wave function () which corresponds to amplitude of the corresponding three dimensional electron waves.

- A linear combination of two AO’s can be achieved by addition and subtraction of two atomic orbitals of the individual atoms.

- If two atoms A & B each containing one electron, their wave functions are represented by A $ B respectively.

- Molecular orbital’s of the covalent molecule AB can be represented by:

b = A + λB

a = A – λB

Where b = bonding molecular orbital

a = Anti bonding molecular orbital

A, B = atomic orbital’s of atoms A & B

Λ = partial ionic character of AB molecule.

BMO   is of lower energy than the atomic orbitals, as the electrons feel greater attraction between them.

- ABMO is of higher energy than the atomic orbitals as the electrons feel less attraction between them.

- In the formation of BMO, the two electron waves of the bonding atoms reinforce each other due to constructive interference. Because of this, repulsion with nuclei is very less and BMO always posses lower energy than AO.

- In the formation of ABMO, the electron waves cancel each other due to destructive interference. Because of this, repulsion with nuclei is high.

- The total energy of two MO’s remains the same as that of two original atomic orbitals

Above fig: Formation bonding (  ) & anti bonding (*) molecular orbital by the linear combination of atomic orbital A&B centered on two atoms A & B.

Molecular orbitals of diatomic molecules

- MO’s give the probability of distribution of e- clouds around a group of nuclei.

- Shape and size of a MO depends upon the shape, size, number and orientation of the combining atomic orbitals.

- Each MO can accommodate a maximum of two electrons with opposite spins.

- The filling of MO takes place according to:

- Aufbau principle: molecular orbitals are filled in order of increasing energy.

- Pauli Exclusion Principle:Two electrons in the same orbital cannot have set of all the four quantum numbers identical. It means maximum no. of electrons in an orbital is two & they must have opposite spins.

- Hund’s rule of maximum multiplicity:The pairing of electrons in the degenerate MO’s does not occur, until each of them has one electron.

The relative energy of molecular orbitals:

- From electrons  1–14 e-

1s<*1s<2s<*2s< π2px = π2py<2pz< π*2px = π*2py<*2pz

- From electrons greater than 14

1s<*1s<2s<*2s<2pz < π2px = π2py<2pz< π*2px = π*2py<*2pz

In the same pair of MO’s (&*, π & π*), the anti bonding molecular orbital is less stable & has high energy than the BMO. Energy of 1s&*1s posses lower energies than 2s&*2s due to the formers (1s&*1s) are obtained from 1s atomic orbitals (of lower energy), while the latter are formed from 2s–atomic orbital (of higher energy). Degenerate molecular orbital’s have similar energies i.e., π 2px = π2py& π*2px = π*2py

*2p

π*2p						     π*2p

π2p						2p

2p						     π2p

*2s						*2s

2s						2s

(A)			(B)

- s–p  interaction are neglected  ( O2&F2)

- After applicable  s-p  interactions (N2)

Molecular orbital energy level diagram of Nitrogen (N2), Oxygen (O2) and Fluorine (F2):

1) Nitrogen Molecule (N2): 14e- : 1s22s2*2s2   π2py2 π2px22pz2

From the electronic configuration of Nitrogen, it has ten electrons in bonding molecular orbitals and four electrons in anti-bonding molecular orbitals.

Therefore, it’s bond order is ½ (Na–Nb) = (10–4)/2  = 6/2 = 3

So, three bonds present between two nitrogen atoms in Nitrogen molecule. It has completely filled HOMO (Highest Occupied Molecular Orbital). It is diamagnetic in nature.

Oxygen Molecule (O2): 16 e  : 1s2*1s22s222pz  π22px = π22pyπ12px  π12py

From the electronic configuration of Oxygen, it has ten electrons in bonding molecular orbitals and six electrons in anti-bonding molecular orbitals.

Therefore, it’s bond order is ½ (Na–Nb) = (10–6)/2= 4/2 = 2

So, three bonds present between two nitrogen atoms in Nitrogen molecule. It has completely filled HOMO (Highest Occupied Molecular Orbital). It is paramagnetic in nature.

Fluorine Molecule (F2): 18 e  : 1s2*1s22s22s*22pz  π22px = π22py π*2px2 = π*2py2

From the electronic configuration of Fluorine, it has ten electrons in bonding molecular orbitals and eight electrons in anti-bonding molecular orbitals.

Therefore, it’s bond order is ½ (Na–Nb) = (10–8)/2         = 2/2 = 1

So, one bond present between two fluorine atoms in Fluorine molecule. It has completely filled HOMO (Highest Occupied Molecular Orbital). It is diamagnetic in nature.

Molecular orbital theory of π–systems

A p-orbital consists of two pairs, usually called lobes.

- Positive sign means a positive displacement of the wave (up) and a negative sign means a negative displacement (down)

- The two parts have opposite phase signs.

- The place, where the phase signs change is called a node.

- Nodes are places where the probability of finding the electron is zero (The e- cannot be there)

π–Molecular orbital of 1,3-Butadiene:

Butadiene is composed of four carbons with two adjacent pi bonds & these π bonds are conjugated (i.e. alternate double bonds) since butadiene consists of four individual p-orbitals the π–system of butadiene will contain 4π molecular orbitals (No. of sp2 carbons = No. of π molecular orbitals)

Molecular orbitals of the butadienyl system:

Configuration:  12223040

- The lowest energy molecular orbital (1): Draw four consecutive p–orbitals all aligned the same way. All positive sign lobes are up & all negative sign lobes are down or vice versa. Numbers of nodes are zero.

- The second lowest energy MO (2): The second lowest energy MO is butadiene will have 1 node

- The third lowest energy molecular orbital (3): It has two nodes and they are placed symmetrically with respect to the centre.

- The highest energy MO (4): It is very easy to draw. Just draw ‘n’ p–orbitals & alternate the phases of each. This creates the π–system with three nodes.

- The highest energy level has (n–1) nodes.

- Highest occupied molecular orbital (HOMO) is 2

- Electrons from the HOMO are donated.

- Most available for bonding

- Characteristic for nucleophilic component

- Lowest un occupied molecular orbital (LUMO) is 3

- LUMO receives electrons

- Lowest energy orbital available

- Characteristic for electrophilic component

- HOMO & LUMO are called as frontier molecular orbitals.

- Frontier molecular orbitals of a compound are at the frontier of electron occupation.

π-Molecular orbitals of benzene

- Benzene has a planar hexagonal structure in which all the carbon atoms are sp2 hybridized and all the carbon–carbon bonds are equal in length and has 6 π electrons.

- The delocalization of the p–orbital carbons on the sp2 hybridized carbons gives the aromatic equalities of benzene.

- For linear system (like butadiene), the highest energy level has (n-1) nodes.

- But for benzene or cyclic systems, the (n-1) rule fails, drawing a molecular orbital of benzene with 5 nodes is impossible.

- In case of benzene, we can count six places where the phases change.

- Instead of looking at these nodes, it is more helpful to think of these as three nodal places, which cut through the molecule at various points.

To draw the π MOs of benzene, we can compare it with hexatriene

- In the lowest energy level (1) for hexatriene and benzene, number of nodes are equal to zero.

- Hexatriene with  2 with one node

+

2:

- But it is impossible to draw a cyclic π system with one node, but we can draw a system with one nodal plane.

- We have drawn a nodal plane that cuts through two of the single bonds.

2

Benzene with 1 nodal plane (through bond)

3

Benzene with 1 nodal plane (through atoms)

- 2&3 MO’s have same number of nodal planes and therefore have the same energy.

- Therefore, we can say that 2&3 MO’s are degenerate (same energy).

- This is the main difference between cyclic & a cyclic system.

5						4 nodes

4						3 nodes	     for hexatriene

3						2 nodes

- In benzene, next levels   4&5  have same number of nodal plane

4 – Through bond				5 – through atom

- 4&5   having same energy  & two nodal planes

- In hexatriene,  6 has 5 nodes

- But in benzene , 6 has 3 nodal planes

Nodal plane (1)

Nodal plane (2)

6:

Nodal plane (3)

3 nodes-6

2 Nodes-4                                                                       2 Nodes-5

E						Anti bonding

Bonding

1Node-2    1Node -3

No nodes-1

π – molecular orbital’s of benzene

note :

in hexatrience		   in benzene:

Crystal Field Theory

Salient features of CFT:

- CFT  gives much more satisfactory explanation for the bonding and the properties of the complexes than valence bond theory (VBT)

- According to CFT, the interaction between metal ion and the legend is purely electrostatic i.e., metal & ligand bonds are 100%  ionic in nature

- Negative ligands are treated as point charges and neutral ligands are treated as dipoles. Thus the bonding in the complex may be ion–ion interaction or an ion-dipole interaction.       Point charges

L

MN+

- Ion – Ion interaction

M

L

Dipoles

--- ion – dipole interaction.

- The five d–orbitals in an isolated gaseous metal atom or ion are degenerate i.e. all have the same energy.

- When a ligand approach the metal ion to form a complex the electrons in the d–orbital of the metal will be repelled by the negative charge or lone pair electrons of the ligand.

- As a result, the energy of the d–orbitals increases and the degeneracy of the d–orbitals is lifted

- The pattern of splitting of the d–orbitals depends on the number of ligands and their arrangement around the central metal atom or ion.

d – orbitals

dxy, dyz, dxz,dx2-y2,dz2

All five orbitals are degenerate (equally energetic) orbitals in free metal ion.

The three orbital’s (dxy, dyz & dxz) are in between the axis called as non axial orbitals.

dx2–y2& dz2 are degenerate & are along the axis called as axial orbitals.

- dxy, dyz and dxz are triply degenerated & represented by t2g

- dx2-y2& dz2 are doubly degenerated & represented eg

- In the absence of ligands all the d–orbitals are in degenerate state.

- In the presence of ligand field, d–orbitals are splits into the groups. This is called crystal field splitting (CFS)

- CFS depends on nature of metal & nature of ligand.

- The energy gap between low energy d–orbital to high energy d–orbital is indicated by  (or) 10 Dq.

- Splitting of d–orbitals decreases the complex energy. This is called crystal field stabilization energy.

- The absolute units of CFSE is cm-1

- CFSE depends on geometry, number of d–electrons, spin pairing energy and ligand character (via spectro chemical series)

Splitting of d–orbitals in octahedral field:

- The number of ligands surrounded by the central metal ion decides the geometry of the complex.

- Suppose MLn+6  is a octahedral complex compound

Where M =central metal on (d–block or transition elements)

- In octahedral complex, dx2-y2 and dz2 orbitals are co-axial with ligands

- Two lobes of dx2-y2 are on the x–axis & two lobes are co-axial with four ligands (L)

- Similarly, two lobes of dz2 are on the z–axis are co axial with two ligands (L)

- In octahedral field, first ligands approach the central metal ion in a spherical field. Due to this, all the d–orbitals energy increases. This energy state is called barry centre.

- In this state, the orbitals which are present along the axis experience more repulsion and hence their energy is increases, from the barry centre.

- The orbital’s which are present in between the axis (dxy, dyz and dzx) decreases the energy from the barry centre

(eg – ‘e’  refers to doubly degenerated, and in t2g – ‘t’ refers to triply degenerated and ‘g’ refers to gerade (can be applied for center of in version of octahedron)

- Energy gap between t2g & eg orbitals is indicated by o

- T2g orbitals energy decrease by -0.4 o& eg orbitals energy increase by 0.6 o.

- This is called CFSE of octahedral complex.

= energy difference

O = refers to octahedral structure

= energy difference        t = refers to tetrahedral structure

Dq = differential of quanta implying the energy.

CFS is the energy difference between t2& eg orbitals

Spectrochemical series: A spectrochemical series is a list of ligands ordered on ligand strength.

I-< Br-< S-2< SCN-< Cl-< N-3< F-< OH-< C2O4-2< H2O < NCS-< NH3< en < CN-< CO

Metal ions with 4 – 7 electrons in the d orbital can exist as high-spin or low-spin complexes. Weaker ligands tend to give high-spin complexes, whereas stronger ligands tend to give low-spin complexes.

Splitting of d–orbitals in tetrahedral complexes:

- In tetrahedral geometry, four ligands are arranged at the alternate corners of the cube and the central metal ion present at the centre of the cube.

= energy difference        t = refers to tetrahedral structure

Dq = differential of quanta implying the energy.

CFS is the energy difference between t2& e orbitals

- Three coordinate axis are present at the centre of the cube faces.

- None of the ligands do approach the metal ion directly in tetrahedral geometry. But the orbitals which are present in between the axis are closed to the ligand field. Hence these orbitals energy increases and orbitals which are present along the axis, the energy decreases.

- In tetrahedral ‘g’ subscript is omitted because they does not contain centre of symmetry.

- In tetrahedral, ‘e’ orbitals (dx2–y2& dz2) energy decreases by – 0.6 t and ‘t2’ orbitals energy increases by 0.4 t.

- Tetrahedral splitting is less compare to octahedral splitting i.e., t = 49o because

- None of the ligands does approach the metal orbitals directly.The number of ligands i.e, in octahedral has the more negative charge due to more electron, therefore more repulsion.

Splittig of d–orbitals in square planar complexes:

- In these complexes, the central metal ion is dsp2 hybridized.

- Square planar co ordination can be imagined to result when two ligands on the z–axis an octahedron are removed from the complex,leaving only the ligands in the x–y plane.

- As the z–ligands move away, the ligands in the square plane move a little closer to the metal.

- The orbital splitting diagram for square planar co ordination can be derived from the octahedral diagram.

- As ligands move along the z–axis, d–orbitals with z–component will fall in energy.

- The dz2 orbital falls the most, as its electrons are concentrated in lobes along the z–axis.

- The dxz orbital also drop in energy, but not as much.

- The dx2-y2 and the dxy orbitals increase in energy.

- The splitting diagram for square planar complexes is more complex than for octahedral and tetrahedral complexes.

In t2g orbitals, dxy orbital energy increases and dxz & dyz orbital energy decreases.

- In eg orbitals dz2 orbital energy decreases and dx2-y2 orbital energy increase.

- Splitting of square planer complexes is more compare to octahedral because the four ligands are strongly attracted by the central mutation due to presence of less e- density along axis. This causes large splitting.

sp  = 1.3 o

sp  >o  >>t

Sp means square planar structure.

Distortion in octahedral complex: (John–Tellar–theorem)

- If all the metal ligand bonds are equal, it is called regular octahedral.

- If all the metal ligand bonds are not equal, it is called distorted octahedral complex.

- The distortion is due to uneven or unsymmetrical filling of electrons.

- Unsymmetrical filling of t2g orbital causes less distortion because these orbitals are present along the path of ligands.

Calculation of CFSE:

CFSE is theoccurence of energy due to splitting of d–orbitals

CFSE of octahedral complex = ( - 0.4 p + 0.6q) o +mP

P = no. of  es in t2g orbitals

q = no. of es in eg orbitals

m = no. of es

P = pairing energy

CFSE of tetrahedral complex = ( -0.6 p + 0.4q) t + mP

P = no. of es in ‘e’ orbital

q = no. of es in ‘t2’ orbital

UNIT-II:

ELECTROCHEMISTRY & BATTERIES

INTRODUCTION: Electrochemistry is the branch of physical chemistry which deals with the transformation of electrical energy into chemical energy and vice versa. Electrolysis, electroplating etc. use electrical energy to drive chemical reaction. While chemical energy is transformed into electricity in galvanic cell, batteries and fuel cells.

Electrochemical Cell :    [Galvanic Cell or Voltaic Cell]

An electrochemical cell is a device in which a redox reaction is utilized for the conversion of chemical energy into electric energy.

Ex : Dry Cell, and Lead storage battery

The electrode where oxidation occurs is called anode; while the electrode where reduction occurs, is called cathode.

CONSTRUCTION OF DANIEL CELL:

The practical application of an electrochemical or galvanic cell is Daniel Cell. Its consists of zinc electrode, dipping in ZnSO4 solution (1M) and a copper electrode, dipping in CuSO4 solution (1M) taken in two different beakers.

The two solutions are connected by means of a salt bridge. Salt bridge is an inverted U – tube containing an aqueous solution of electrolyte such as KCl, KNO3, or K2SO4 (which does not react chemically during the process). Some gelatin (or) agar – agar is added to this aqueous solution of electrolyte to converted it into semi solid paste.

The moment the two electrodes are connected by a wire, the following reactions occur at different electrodes.

At anode 				Zn 2+(aq)  + 2e- [oxidation half reaction]

At anode 	Cu2+(aq)+2e-		 Cu(s) [reduction half reaction]

+ Cu2+(aq)			Zn2+(aq) Cu(s) [ complete cell reaction]

Oxidation, takes place at the anode and reduction taken place at the cathode. The electrons flow from anode to cathode. The transference of electrons from Zn anode to Cu cathode leads to the accumulation of positive charge around the anode due to the formation of Zn2+ ions and the accumulation of negative charge around cathode due to deposition of Cu2+ ions as copper on the cathode. The positive charge so accumulated around the anode will prevent the flow of electrons from it. Similarly, accumulation of negative charge around the cathode will prevent the acceptance of electrons from the anode. As the transference of electrons stops, the current in the electrical circuit also stops.

All this stage, the salt bridge comes to the aid and restores the electrical neutrality of the solution in the two half cells (electrodes). When the concentration of Zn2+ ions around the anode increases, sufficient numbers of C1- ions migrate from the salt bridge to the anode half cell. Similarly, sufficient numbers of K+ ions migrate from the salt bridge to cathode half cell for neutralizing excess negative charge due to the additional ions in the cathode half cell. Thus, salt bridge provides cations and anions to replace the ions lost or produced in the two half cells. It maintains electrical neutrality of the two solutions in the two half cells. It allows the flow of current by completing the electrical circuit. It prevents intermixing a solution as well as ions in the two half cells, which may lead to a precipitation reactions.

Representation of Galvanic cell:

Zn / Zn SO4(aq) || CuSO4(aq) / Cu

Zn ; Zn+2 (1M)|| Cu2+ (1M); Cu

- on the left hand side anode is written and on the right hand side cathode is written.

- Anode is written by writing the metal first and then the electrolyte. The two are separated by vertical line or semicolon.

- Cathode is written by first writing the electrolyte and the metal thereafter. These two are separated by vertical line (or) semicolon.

- The electrolyte may be represented as complete formula of the whole compound (or) by ionic species.

- The two half cells, anode and cathode, are separated by double vertical lines which indicate “salt bridge”.

- Electromotive Force (OR) Cell potential (EMF)

The flow of electricity from one electrode to another electrode in a galvanic cell indicates that the two electrodes have different potentials. This difference of potentials between the electrodes of a cell which causes flow of current from an electrode at higher potential to another at lower potential is known as electromotive force (or) cell potential.

The EMF of a cell is a qualitative measurement of the tendency of a net reaction to occur within the cell. Since the net cell reaction is a redox reaction, the EMF of a cell in a way measures relatively the tendency of oxidation reaction to occur at the anode and the tendency of reduction reaction to take place at the cathode of the cell:

Mathematically :

EMF or E Cell = oxidation potential of the anode + Redunction potential of the cathode

Since oxidation takes place at anode and reduction takes place of cathode.

E Cell	= E anode (oxidation) + E cathode (Reduction)

and E Cell =E cathode (Reduction) - E anode (Reduction)

E Cell  = oxdn potential of anode – oxdn potential of cathode

Always remember that when potential is not specified (i,e) as oxdn or redn potential; it means the given value is for reduction potential. While solving numerical problem, sign should be given importance. If Cell potential (EMF) is positive, the reaction is spontaneous. Further, the more +ve value of the E cell, the more faster is the reaction.

Numerical problemson emf

Electro chemical series: When all single electrodes are arranged in increasing order of their standard reduction electrode potentials, a series is obtained called electro chemical series.

Applications of electro chemical series:

- Relative strength of oxidizing and reducingstrength:

The electro chemical series helps in predicting the oxidizing and reducing ability of a substance. In electro chemical series the elements are arranged in the order of their increasingreduction potential, hence the elements situated at the bottom of the series have a greater tendency to get reduced—they are good oxidizing agents. On the other hand, elements at the top of the series have low reduction potential and tendency to get oxidized—they are good reduction agents.

- Predicting spontaneity or feasibility of areaction:

For a cell reaction to be spontaneity, the EMF of the cell should be positive. If E0cell is negative then the cell reaction with not be feasible.

- To predict whether a metal will react with acids to give H2gas:

Those metals whose reduction potential is less than the reduction potential of hydrogen can liberate H2 gas. Such metals are placed above hydrogen in the electrochemical series. Metals placed below hydrogen in the electrochemical series have a higher value of reduction potential than hydrogen. Such metal do not liberate hydrogen on reacting with acids.

- Replacement tendency:

The metal at the top of the electrochemical series has the tendency to replace the one below it from its solution. This is because the metal placed at top has a lower reduction potential and hence a greater tendency to donate electrons.

NERNST EQUATION: -

The equation which gives relation between electrode potential and its ionic solution concentration is called Nernst equation.

Consider the following redox reaction

M  ↔ Mn+ + ne-

For such a redox reversible reaction, the free energy change (ΔG) and its equilibrium

ΔG = ΔG0+ RT ln [product]/[Reactant]……………Eq- (1)

Where ΔG= free energy change

ΔG0 = standard free energy change

The above equation is known as Van’t Hoff Isotherm. The decrease in free energy in the reversible reaction will produce electrical energy i.e.

-ΔG = nEF and

-ΔG0 = nE0F ……………Eq-(2)

Where E = Electrode potential

E0 = Standard electrode potential

F = Faraday (96,500 coulombs)

Comparing equation 1 & 2

-nEF = -nE0F + RT ln [M]/[Mn+]

= -nE0F + RT ln 1/[Mn+]

Where, concentration of the metal is unity or

-nEF = -nE0F - RT ln [Mn+]

Dividing the equation by –nF

E= E0 + RT ln [Mn+]

nF

E= E0 + 2.303RT log [Mn+]nF

E= E0 + 0.0591 log [Mn+] ……………Eq-(3)

n

This equation is known as “Nernst Equation” for electrode potential

TYPES OF ELECTRODES

The electrodes which are used to calculate the other electrode potential values are called reference electrodes. The best reference electrode used is standard hydrogen electrode where electrode potential at all temperatures is taken as zero.  However in view of the difficulties in setting up of a hydrogen electrode other electrodes are also used as reference electrodes.

(1) . CALOMEL ELECTRODE: -

This is commonly used as a secondary reference electrode for potential measurements in place of a hydrogen electrode. The calomel electrode consists of mercury in contact with mercurous chloride and a solution of soluble chloride ions such as KCl of known concentration. It is represented as.

Pt, Hg / Hg2Cl2(s), KCl (solution)

CONSTRUCTION

Mercury is placed at the bottom of a glass tube having two side tubes on each side. Mercury is covered by a paste of Hg2Cl2.Asolution and KCl (decinormal or saturated) solution is introduced above the Hg2Cl2 through the side tube. A Pt wire sealed into a glass tube serves to make electrical contact of the electrode with the circuit.

Net cell reaction is

Hg2Cl2 + 2e-  ----------------- 2Hg + 2Cl-

The electrode potential is given by

E= E0 + 0.0591 log [Cl-]2	       2

or

# E= E0 + 0.0591 log [Cl-]

# 2

The standard reduction potential of calomel electrode at 25oc is

For saturated KCl solutions potential is 0.2415 V

For 1N solutions potential is 0.28 V

For 0.1 N solution is 0.3338 V

2. QUINHYDRONE ELECTRODE: - This is a redox electrode reversible to protons and often replaces the hydrogen. Quinhydrone is a 1:1 molar mixture of quinine and hydroquinone. The electrode consists of a platinum electrode dipped in an acid / base test solution, which is saturated with quinhydrone. The electrode reaction is given by

Quinine(Q)				hydroquinone(QH2)

The electrode potential at 25˚c is given by

Since quinone and hydroquinone are taken in equimolar amounts

i.e., [Q] = [ QH2]

This equation can be used in the calculation of PH if Eo and E values are known. The standard electrode potential Eo value is – 0.006944 v and for determination of E, the quinhydrone is coupled with calomel electrode and EMF of cell is measured

Advantages: The quin-hydrone electrode is simple to set up and needs no removal of air. The reversibility equilibrium is achieved faster than hydrogen gas electrode thereby allowing a quicker measurement. pH values of solutions containing reducible substances like Cu2+, Cd2+, unsaturated acids, NO3-, etc., and catalytic poisons can be measured using quin-hydrone electrode, where hydrogenelectrode cannot be used.

Limitations: The electrode cannot be used at pH values greater than 8. Even this electrode fails in the presence of strong oxidizing and reducing agents.

3. IONE SELECTIVE ELECTRODE: -

Ion selective electrodes (ISE) use a membrane which is sensitive to a particular chemical species. ISE respond to certain specific ions present in a mixture while ignoring others and develop potential. In these electrodes a semi permeable membrane separates two solutions of a salt at different concentrations. The semi permeable membrane permits the passage of only one type of specific ion, thereby developing a potential with respect to that species. A typical example of this type is the glass electrode which is selective to H+ ion.

GLASS ELECTRODE: -

Glass electrode is widely employed electrode for pH measurements by combining this electrode with calomel electrode. It is based on the principle that when two solutions of different H+ ion concentrations are separated by a glass membrane, a potential difference develops across the glass membrane. The magnitude of the potential difference depends upon the difference in the concentration of the hydrogen ions in the two solutions. The glass membrane is a special type of membrane having the approximate composition of 72% SiO2, 22% Na2O and 6% CaO.

A glass electrode consists of a thin walled bulb of pH sensitive glass sealed to a stem of non

PH sensitive high resistance glass. Inside the glass electode is a Ag/AgCl electrode or  a Pt wire is dipped in 0.1 N HCl solution.

The magnitude of potential difference is given by

EG = E0G + 0.0591 log [H+]      or

EG = E0G - 0.0591pH

Where EG is the potential of electrode.

E0G is standard potential of glass electrode

The E0G value of glass electrode can be determined by using a solution of known pH

This glass electrode is dipped in a solution of varying hydrogen ion concentration and is connceted to an exrnal reference electrode, usually a calomel elecyorode.the cell is represented as.

Ag,AgCl, 0.1 HCl/Glass/External solution/KCl,Hg2Cl2/Hg,Pt

Ecell=Ecalomel0-EGlasselectrode

Ecell=0.2422-(EG0-0.0591 PH)

Ecell=0.2422-EG0+0.0591 PH

Ecell+ EG0-0.2422=0.0591PH

PH=Ecell+ EG0-0.24220.0591

Advantages of Glass Electrode

- It is simple and can be easily used

- Equilibrium is rapidly achieved.

- The results are accurate

Limitations of Glass Electrode

- The glass electrode can be used in solutions with pH range of 0 to 10. Electrodes composed of special glasses can be used for measurements up to a pH of 12

- Although glass membrane of electrode is very thin, yet its resistance is extremely high (of the order of 10 – 100 million ohms), which cannot be measured by ordinary potentiometers. It is, therefore, necessary to use special electronic potentiometers.

BATTERIES

When two  or more electrochemical cells are electrically interconnected in series that can be used as a direct electric current at a constant voltage. These are used in commercial purpose and divided into threetypes.

(1)Primarycells	(2) secondarycells

- PRIMARY CELLS:-These are non-rechargeable cells. In these a chemical reaction takes place spontaneously and the free energy change of the reaction is converted into electrical energy and once the reactants are exhausted, production of electrical energy also stops. Thechemical

reaction cannot be reversed by passing electricity through the cell hence a discharged cell cannot be used again.

Lithium Batteries.

Lithium batteries are primary batteries that us lithium metal or lithium compounds as anode. They stand apart from other batteries in their high charge density and long life. Depending on design and chemical compounds used, lithium cells can produce voltages from 1.5 to 3.7 v.

Primary lithium batteries are further classified on the basis of the type of electrolyte or solvent and material of cathode. The basic types are soluble cathode, solid cathode and solid electrolyte.

Ex Li-MnO2 Battery. It is belongs to solid cathode battery. In this battery, lithium is used as the anode, while a spatially prepared heat-treated MnO2 is used as cathode. Electrolyte is lithium salt ( lithium pechloride) in a missed organic solvent propylene carbonate and dimethoxy ethane.

During the reaction, Mn+4 is reduced to Mn+3 by the interstitially occupied lithium ions in the MnO2 intercalation compound. LiMnO2 indicates the Li+ ions remain present in the host MnO2 lattice.

The cell is represented as

Li/LiClO4 in propylene carbonate + Dimethoxy ethane/MnO2 Atanode	Li	Li+ +e-

Atcathode	Li++ e-+MnO2	LiMnO2

Overall	Li+MnO2	LiMnO2

It is used in automatic cameras, calculators and watches.

- SECONDARYCELLS:-

These are rechargeable cells. After every use the electrochemical reaction is be reversed by external application of current. These are meant for multi cycle use. When it supplies electrical energy, it operates like electrochemical cell. During recharge it operates like electrolytic cell.

Ex: - (I) LEAD ACID CELL:-

The lead storage battery is one of the most common batteries that are used in the automobiles.

It contains a grid of lead- antimony alloy coated with PbO2 cathode and spongy lead as anode. The electrolyte is a 20% H2SO4 solution. These electrodes are arranged alternately, separated by a thin wooden piece and suspended in dil. H2SO4.

anode	Pb

Cathode	PbO2 Electrolyte	20%H2SO4

Discharging :

When battery is used for supplying electrical energy, it is said to be discharged.in the process, lead electrode gets oxidized to Pb+2 ions which then combine with SO42- ions to form PbSO4.

Pb	> Pb2+ + 2e- (at leadanode)

Pb2++SO42	>PbSO4

The electrons released from the lead anode flows to the PbO2 electrode. Here PbO2 gains electrons to form Pb2+ ions.

PbO2  + 4H++2e	> Pb2+ + 2H2O (at PbO2cathode)

Pb2++SO42	>PbSO4

So the net reaction during use or discharging is,

Pb + PbO2 + 4H++2SO42	> 2PbSO4+ 2H2O +Energy

It may be noted that PbSO4 is precipitated at both the electrodes. The voltage of each cell is about 2.0 volts at a concentration of 21% H2SO4 at 250c. Lead cell, commonly used in automobiles, is a combination of six such cells in series to form a battery with an EMF of 12 volts.

Charging

During use PbSO4 precipitated at both the electrodes. When PbSO4 covered completely both the electrode, the cell stops working. For further use it needs to be recharge. The cell can be recharged by connecting it to an external direct current source so as to reverse the cell reaction. The +ve pole of the generator is attached to the +ve pole of the cell (or) battery and the following reactions takeplace

2PbSO4  + 2H2O+Energy	> Pb + PbO2 +2H2SO4

During the process of the charging, the electrodes of cell are restored to their original conditions (i.e. Pb and PbO2 respectively).

Uses

Lead acid storage cells are used for many purposes such as to supply current for electrical vehicles, gas engine ignition, in telephone exchanges, railway trains, mines, laboratories, hospitals, broadcasting stations, automobiles, power stations.

2) Lithium ion batteries:-

Lithium-ion battery (LIB) is a type of rechargeable batteryin which lithiumionsmove from the negative electrodeto the positive electrode during discharge and back when charging.

The three primary functional components of a lithium-ion battery are the positive and negative electrodes and electrolyte. Generally, the negative electrode of a conventional lithium-ion cell is made from carbon. The positive electrode is a metal oxide, and the electrolyteis a lithium salt in an organicsolvent.

The most popular negative electrode is graphite.

The positive electrode is generally one of three materials: a layered oxide (such as lithiumcobalt oxide), a polyanion (such as lithium iron phosphate) or a spinel (such as lithium manganeseoxide).

The electrolyte is typically a mixture of organic carbonates such as ethylene carbonateor diethyl carbonatecontaining complexesof lithium ions.

Reactions

Atanode	LiC6	--------Li+ + e- +C6

Atcathode	Li1-XCoO2 + Li+ + e- --------Li1-XCoO2

In lithium ion battery the lithium ions are transported to and from the positive or negatie electrodes by oxidizing the transition metal, cobalt, in Li1-XCoO2 from  during charge Co3+ to  Co+4. The cell's energy is equal to the voltage times the charge. Each gram of lithium represents Faraday'sconstant/6.941or13,901coulombs.At3V,thisgives41.7kJpergramoflithium,or

11.6 kWh per kg.

APPLICATIONS OF BATTERIS:-

- These act as portable source of electricalenergy.

- A variety of electronic gadgets have been made more useful and popular with the introduction of rechargeable storagebatteries.

- For all commercial applications, batteries are constructed for ther service, for example batteries for automotives and aircrafts, standby batteriesetc.

UNIT-IV:

Fuels & Combustion

Fuels & Combustion

Energy is a part of life. Better energy sources are status symbol of a country. Fuels are mainly the energy sources for the industry and domestic purposes. For many years to come, the greatest portion of the world’s power will come from the combustion of fuels.

Fuel is a combustible substance, containing carbon as main constituent, which on proper burning gives large amount of heat, which can be used economically for domestic and industrial purposes.

Ex: Wood, Charcoal, Coal, Kerosene, petrol, Diesel, Producer gas, Oil gas etc… are some of the fuels.

The combustion is a quick, high temperature oxidation of fuels, converting carbon to CO2 and hydrogen to water with the evolution of heat and light. During the process of combustion of a fuel (like coal) the atoms of carbon, hydrogen, etc. combine with oxygen with the simultaneous liberation of heat at a rapid rate. This energy is liberated due to the “rearrangement of valancy electrons”, in these atoms, resulting in the formation of new compounds (like CO2, H2O etc.). These new compounds have less energy (or heat content) in them and, therefore, the energy (or heat) released during the combustion process is the difference in the energy of the reactants (C, H and O etc; of fuel) and that of the products formed.

Fuel + O2                 Energy + Products

The main source of fuels is coal and petroleum oils, the amounts of which are dwindling day –by –day. These are stored fuels available in earth’s crust and are, generally called “fossil fuels.

Now a day’s different type of fuels are available

Fuels

Bio fuels 				        fossil fuels				   nuclear fuels

[biomass (soild) bio gas (liquid)      [petroleum products](Liquid)          [92U 235,]

U=uranium

Pu= plutonium

Classification of fuels:-

Classification of fuels is based on two factors .

- The way it is obtained (natural or artificial)

- Physical state of the fuel (solid, liquid, gas).

Based on above said factors, fuels are classified into 2 types

- Natural fuels (or primary fuel):-

Some fuels are found in nature and are used in the some form are called natural fuels, or primary fuels.

Ex:-wood, coal and natural gas.

- Artificial fuel (secondary fuel):-

Some of the fuels that are derived from other fuels (primary) are called artificial or secondary fuels.

Ex:- petrol and charcoal.		Fuel

Natural (primary) fuel 					artificial(secondary) fuel

Solid       liquid	     Gas				  Solid              liquid	                 Gas

Comparison of solid, liquid and gaseous fuels :-

| Solid | Liquid | Gas |
| --- | --- | --- |
| Cheap and easily available | Costly and available only in the Arabic countries and obtained from mines | More costly except natural gas |
| As it does not burn spontaneously , its storage transportation and use is easy | Transportation easy and storage needs care | Transportation is easy but storage is risky |
| Fire hazards are least | More risky | More risky |
| Slow combustion | Quick combustion | Very fast combustion |
| Ash content is more | No ash content | No ash content |
| Causes more pollution | Less pollution | Least pollution |
| Low calorific value | Higher calorific value | Highest calorific value |
| More oxygen is required for combustion | Less O2 is required for combustion | Least O2 is required for combustion |
| It cannot be used in vehicles as fuel | Mainly used in vehicles as fuel | Also used as fuel for vehicles |

CALORIFIC VALUE :-

The calorific value of any fuel is a very important property. It measures the heat produced by the fuel. Higher the calorific value better will be the fuel.

It is defined as “the amount of heat produced by the combustion of unit mass or unit volume of a fuel”.

The calorific value is measured in several units of heat; they are calorie, kilocalorie, British thermal unit (B.T.U) and centigrade thermal units (CTU) or Centigrade Heat Unit (CHU). Relationship amoung all the above units of heat is given below.

1 kcal = 1000 cal = 3.968 BTU = 2.2 CHU  (or) CTU

Joule is also a unit of heat or energy.

1 Cal = 4.18 Joule.There are two types of calorific values of a fuel

- High Calorific Value ( HCV) or Gross Calorific Value (GCV)

- Low Calorific Value (LCV) or Net Calorific Value (NCV)

High Calorific Value ( HCV) or Gross Calorific Value (GCV) :-

It is defined as “the total amount of heat produced, when unit mass or unit volume of the fuel has been burnt completely and the products of combustion have been cooled to room temperature.”

Usually, all fuels contain some hydrogen and when the calorific value of hydrogen – containing fuel is determined experimentally the hydrogen is converted into steam. If the products of combustion are condensed to the room temperature, the latent heat of condensation of steam also gets included in the measured heat, which is then called HCV or GCV.

2 .Low calorific value (or) net calorific value :-[ LVC (or)NCV]

It is defined as the net heat produced, when unit mass or unit volume of the  fuel is burnt completely and the products are permitted to escape.

In actual use of any fuel, the water vapor and moisture, etc, are not condensed and escape as such along with hot combustion gases. Hence a lesser amount of heat is available

Relationship between HCV and LCV :-

H2 +   O2           H2O

If hydrogen is present in a fuel, the above –mentioned chemical reaction will takes place and 2g of hydrogen will produce 18g of H2O (or) 1g of hydrogen will produce 9g of H2O. if x g hydrogen is present in a fuel, it will produce 9x gr of water [ because 1 part by mass of hydrogen produces 9 parts by mass of water ]; and 9x gr of water vapour that will release 9x  2 cal/g heat on cooling,
[cal/g is the latent heat of water vapour].

So ,

LCV=HCV – latent heat of water vapour formed (or)

LCV= HCV- 9 X L

Further,

HCV=LCV + 9L 		     [ in case hydrogen is Y%, the above relation will be;

HCV= LCV+ 0.09 h 587, LCV=HCV-0.09H 587

X= mass of hydrogen ;L= latent heat of water vapour, the latent heat of steam is 587 kcal/kg (or) 1,060 B.Th.u/lb of water vapour at R.temp.(room temp)

It is generally expressed in calorie/gram (cal/g) or kilo calorie/gram kilo  (k cal/kg) or British thermal unit/lb (BTU/lb) in case of solid or liquid fuel.

In case of gaseous fuels, the unit used is kilo calorie /cubic metre (K cal/m3) or but/cubic feet (BTC/ft3).

Characteristics of a good fuel:-

A good fuel has the following features:

- It should be easily available

- It should be dry and should have less, moisture content, dry fuel increases its calorific value.

- It should be cheap, poor people cannot afford costlier fuel.

- It should be easily  transportable ,otherwise cost of fuel will increase

- It must have high calorific value .

- It  must have less ash after combustion. In case  more ash, the fuel gives less heat.

- It must have moderate temperature; low burning (or) ignition temperature can cause fire accident.

- It should not burn spontaneously to avoid fire hazards.

- It should not give poisonous gases after combustion

- Its handling should be easy.

- The combustion of a good fuel should not be explosive.

Determination of calorific value by Junker’s Gas Calorimeter

Calorific value of solid and liquid fuels is determined by “Bomb-Calorimeter”. The calorific values of gaseous fuels are determined by Junker’s Method.

Determination of calorific value by Junker’s gas calorimeter:- Calorific value of solid and liquid fuel are determined by “Bomb calorimeter. The calorific value of gaseous fuels is determined by Junker’s method. This method can be applied to determine CV (calorific value) of liquid fuel also which are easily vaporized. This calorimeter works on the junker’s principle. According to which.

----------------------------------------------------------------------------------------------------------------          C V of gaseous fuel   vol.of gas = vol.of water  rise in temp. of water

----------------------------------------------------------------------------------------------------------

The main parts of Junker’s calorimeter are described below:

- Bunsen burner is a special type of burner clamped at the bottom. It can be pulled out of the combustion chamber (or) pushed up into the chamber during carrying out of combustion.

- Gasometer is   employed to measure the volume of gas burning per unit time. This is attached with a Nano meter fitted with a thermo meter. So that pressure and temperature of the gas before burning can be read.

(iii) Pressure governor can control the supply of quantity of gas at a given pressure. It covered with at floating drum over the water. Pressure governor assists in maintaining the standard conditions of saturation of fuel gas with water.

(iv) Gas calorimeter has a vertical cylinder (called combustion chamber), which is surrounded by an annular space for heating water and interchange coils. The entire is covered by an outer jacket (of chromium) in order to reduce the heat loss by radiation and convection. Thus, the outer jacket acts as an insulator. The annular space for water heating is connected as its lower end to internal supply of water. Two thermometers are used to record the temperatures of inlet water and outlet water respectively.

During the process of up and down movement through chamber, the following water takes away practically whole of the heat of burner and any steam formed during combustion gets condensed as water, which can be collected.

Junker’s Gas Calorimeter

Working :-

Circulation of water and burning of gaseous fuel are continued at constant rates for about  15 to 20 minutes for initial warming up period. Then the rates of gaseous fuel burning and water circulation are controlled so that products of combustion leaves the apparatus nearly at atmospheric pressure (I.e., 760 mm Hg). When the steady conditions are established, then readings are taken simultaneously of:

- The volume of gaseous fuel burnt (V) at given temperature and pressure in a certain period of time (t);

- The quantity of water (W kg) passing through the annular space during the same   interval of time;

- The steady rise in temperature (T2 – T1), and

- The mass of water (steam) condensed in the outlet water

Calculations: -

Let V =  Volume of fuel gas burnt at STP in a certain time, t;

W = mass (quantity) of cooling water used in time, t;

T1 = Temperature of inlet water

T2 = Temperature of outgoing water

m = mass of steam condensed in time “t” in a graduated cylinder

E = Higher Calorific Value (HCV) of fuel

Then,

Heat absorbed by Circulating water = W (T2 – T1)

And heat produced by combustion of fuel = VE

Assuming no heat loss, we get:

VE = W (T2 – T1)

(Or) HCV or GCV; E = W (T2 – T1) /V

Mass of H2O condensed per m3 of gas = m / V Kg

: Latent heat of steam per m3 of gas = m X 587 / V k. cal

:LCV or NCV = [ E – m X 587/v] K cal /m3

[: E = HCV (or) GCV]

SOLID FUELS: - (COAL)

WOOD:- Wood is obtained from forests. Freshly cut wood contains 25 to 50% moisture, which is reduced to about 15% on air-drying. The average composition of wood on dry moisture-free-basis is: Carbon = 55%, Hydrogen = 6%; Oxygen = 43%, ash = 1%. The calorific value of air-dried-wood is about 3,500 to 4,500 kcal/kg.

COAL: -

Because of the environmental hazards, the trees were buried inside the earth. By the temperature, pressure and bacterial actions, they were converted into a brown-black solid named coal, in thousands of years.

Temperature / Pressure

Wood    ------    --- 			     Coal

Bacterial action

The process of conversion of wood into coal is called “Coalification”. Depending upon the percentage of carbon, hydrogen, moisture and calorific value, four different types of coals exist. It is called “Ranking of coal”

Wood  Peat   Lignite   Bituminous coal    Anthracite

This progressive transformation of wood to anthracite results in :

- Decrease in the moisture content.

- Decrease in hydrogen, oxygen, nitrogen and sulphur contents, with a corresponding rise in carbon content.

- Decrease in volatile matter content.

- Increase in the calorific value.

- Increase in hardness.

- Peat: - Peat is brown fibrous jelly-like mass. It is regarded as the first stage in the coalification of wood. Peat is mostly dug by manual labor. It is uneconomical fuel, since it may contain as much as 80-90% water, but on air-drying (after 1 to 2 months drying), it burns freely. The average composition of air-dried peat is C = 57%, H = 6%, O = 35%, ash content = 2.5-6%.

Its calorific value is about 5,400 kcal / kg.

- Lignites : - (Brown Coals)

These are soft, brown colored variety of lowest rank coals, which consist of vegetable matter decomposed more than that in peat. It consists of 20-60% of moisture and on air-drying; it breaks up into small pieces.  Air-dried Lignite contains, C = 60-70%, O = 20%.

The calorific value is about 6,500 – 7,100 kcal / kg.

- Bituminous Coals : - (Common Coals):

These are pitch-black to dark-grey coals, which usually soil hands. They show a laminated structure of alternate very bright and dull layers. The common bituminous coals are sub-classified on the basis of carbon content.

- Sub-bituminous Coals: -

These are black in color and more homogeneous and smooth in appearance. Carbon content varies from 75% to 83 % and oxygen from 10% to 20%. The calorific value is about 7000 – 7500 kcal /kg.

- Bituminous coals:- These can show a typically banded appearance and carbon content ranges from 78-90% their calorific value is 8,000-8,500 kcal/kg. Coal of this class is used  in the large quantities in industries for making metallurgical coke, coal gas and for steam raising and  domestic heating.

- Semi- bituminous coals:- These included varieties of bituminous coals that are rich in carbon (90-95%) and have low volatile matter.

Their  calorific value is about 8500-8600Kcal/Kg.

- Anthracite :- It is a class of highest rank coal, containing highest percentage of carbon (92-98%) and has lowest volatile matter and moisture content they are hardest of all kinds of coals, quite dense and lustrous in appearance. Their calorific value is about 8,650-8,700 kcal/kg and ash content is very small (about 3%),

Change in the average composition from wood to anthracite,

| Fuel | Moisture of air-dried sample at 40’c(%) | C | H | N | O | CALORIFIC VALUE |
| --- | --- | --- | --- | --- | --- | --- |
| Wood | 25% | 50 | 6 | 0.5 | 43.5 | 3500 - 4500 |
| Peat | 25% | 57 | 5.7 | 2 | 35.3 | 4125 - 5400 |
| Lignite | 20% | 67 | 5 | 1.5 | 26.5 | 6500 - 7100 |
| S.B.coal | 11% | 77 | 5 | 1.8 | 16.2 | 7000 - 7500 |
| B.coal | 4% | 83 | 5 | 2 | 10 | 8000 - 8500 |
| Semi.B.C | 1% | 90 | 4.5 | 1.5 | 4 | 8500 - 8600 |
| Anthracite | 1.5% | 93.3 | 3.0 | 0.7 | 3.0 | 8650 - 8700 |

Analysis of Coal:-

The analysis of coal is helpful in its ranking. The assessment of the quality of coal is carried out by these two types of analysis.

- Proximate Analysis

- Ultimate Analysis

- Proximate Analysis :-  In this analysis, the percentage of carbon is indirectly determined. This analysis includes percentage of moisture, % of volatile substance, % of ash content and % of carbon.

- Moisture:- A known mass of finely powdered coal is taken in a crucible. It is heated up to 1100C in an electric hot air-oven. The crucible allowed to remain in oven for 1 hour and then taken out, cooled in a desiccator and weighed. Loss in weight is reported as moisture.

% of moisture =      Loss in weight      X    100

Weight of Coal taken

- Volatile matter: - The above sample is taken and heated at 9500C in an electric furnace in the absence of air for 7 minutes. It is then cooled to room temperature and weighed. The loss of weight is reported as volatile matter and is removed from coal at 9500C.

% of volatile matter = Loss in weight due to removal of volatile matter X 100

Weight of Coal taken

- Ash Content: - In this analysis, the above coal, free from moisture and volatile matter, is heated in a crucible at about 7000c in the presence of air. It undergoes combustion and results in the formation of ash. Crucible is cooled to room temperature and weighed. The weight of ash is then determined.

% of ash ==      weight  of ash formed     X    100

Weight of Coal taken

- Carbon :-  Since main component of coal is carbon, it can be determined by subtracting the sum of percentage of moisture , volatile substance and ash content from 100.

% of Carbon = 100 – (% of moisture + % of volatile matter + % of ash)

Importance or Significance of Proximate Analysis:-

Proximate analysis provides following valuable information’s in assessing the quality of coal.

Moisture: - High moisture content in the fuel reduces the calorific values and increases the transport costs; the better is the quality of a fuel.

Volatile matter:- A coal containing  high volatile matter burns with long flame, high smoke and low calorific value, volatile matter also influences the design of furnace since the higher the volatile matter, larger is the combustion space required.

Ash:- It is a useless, non- combustible matter, which reduces the calorific value of coal, ash content also increases the cost of transportation, handling and storage and disposal. It determines the quality of coal .Hence the lesser the percentage of ash, the better is the quality of coal.

Carbon:- The higher the fixed carbon in a coal, the greater is its calorific value.

2. ULTIMATE ANALYSIS:-

This analysis includes percentage of C, H, O, S, N and ash content in coal.

- Carbon and hydrogen determination :-

A known weight (1-2gr) of coal is taken and burnt in an apparatus. The carbon change to CO2 and hydrogen changes to H2O. The vapors of CO2 and H2O are then passed through KOH and CaCl2. The CO2 is absorbed by KOH in the tube while H2O is absorbed by CaCl2, because of the absorption, the weight of KOH and CaCl2 increases, which is measured

C+O2                    Ignition CO2 (44)

H2 +  O2                    Ignition H2O (18)

2 KOH +CO2                   K2 CO3 +H2O

CaCl2 + 7 H2O                  CaCl2 . 7H2O (hydrated calcium chloride)

Percentage of carbon =  100

Percentage of carbon =  100

Significance :-  The  higher the percentage of carbon and hydrogen, the better is the quality of coal and higher its calorific value. Percentage of carbon helps in assessing the rank of coal.

b) Nitrogen determination:-  The estimation of nitrogen is done by Kjeldahl’s method

(i) About 1 gr of accurately weight powdered coal is heated with conc. H2 SO4 along with K2SO4 .

(ii) When clear solution is obtained (i.e., when whole nitrogen is converted into ammonium sulphate) it is treated with excess of NaOH to liberate ammonia.

(iii) The ammonia thus produced is distilled over and absorbed in a known volume of standard H2SO4 solution.

(IV) The volume of unused H2SO4 acid is then determined by back titration with standard NaOH solution [un used H2SO4  means unreacted H2SO4]

Percentage of Nitrogen = VolumeofH2SO4 Normality 1.4Weightofcoaltaken (1gr)

Significance  :- Nitrogen  does not have any calorific value. It has no significance, thus, a good quality coal should have very little nitrogen content.

© Sulphur determination: - A known amount of coal is burnt completely in a bomb calorimeter in a current of oxygen, by which sulphur present in coal is oxidized to sulphates. The ash from the bomb calorimeter is extracted with dil.HCl the acid extract is then treated with BaCl2 solution to precipitate sulphate as BaSO4 which is filtered, washed, dried and heated to constant weight.

Percentage of sulphur = weight of  BaSO4x    32 x  100weightof Coalx 233

[Atomic weight of S=32; molecular wt of BaSO4  = 233]

Significance:-  (a)  sulphur  increases  calorific value . (b) The product of combustion SO2, SO3 have corrosive effect on equipment, and cause air pollution.

(d) Oxygen determination: - The % of oxygen is determined by subtracting the sum of percentage of C, H, S, N and ash from 100.

Percentage of oxygen = 100 – [percentage of C+H+N+S+ash]

Significance: -  Oxygen  content decreases the calorific value of coal. High oxygen – content coals are characterized by high inherent moisture, low calorific value. Increases in 1% oxygen content decrease the calorific value by about 1.7% and hence, oxygen is undesirable. Thus, a good quality coal should have low percentage of oxygen.

Liquid fuel :- ( PETROLEUM)

Petroleum is one of the best primary liquid fuels. It is also known as crude oil. Petrol, diesel, kerosene are main liquid fuels. They are secondary liquid fuels derived from petroleum. These fuels are used for domestic works, auto vehicles and power generation.

The word meaning of petroleum is ‘rock oil’ (petra = rock; oleum = oil; petroleum is dark brown viscous liquid.  Petroleum is a mineral found deep in earth’s crust. It is a mixture of number of hydrocarbons (paraffin’s highly active compounds along with traces of compounds of heavy metals like Fe,Co,Ni,V.

Composition of petroleum is given as: [paraffins = alkanes  olivines = alkenes]

C= 80-87.1%, H = 11.1 – 15.0% S = 0.1 – 3.5%, O = 0.1 – 0.9%, N = 0.4 – 0.9%

Origin of petroleum: - There are two theories to explain the origin of petroleum

          - Carbide theory :- [Inorganic theory (or) Mendeleev’s  Theory ]

Metals inside the earth react with carbon and form metal carbides these carbides are converted to hydrocarbons in the presence of moisture. This theory was unable to explain the presence of nitrogen, sulfur and optical active compounds found in petroleum. This is also called Inorganic or Mendeleev’s theory.

(II)Engler’s theory (or) Organic theory: - According to this theory organic matters, animals, vegetation and marine animals were dead and accumulated in sea. There they were decomposed under high temperature and pressure by anaerobic bacteria to give a dark viscous liquid called petroleum. This theory is better accepted.

Classification of crude oil:- (or) petroleum  There are three types of petroleum.

(III)Paraffinic base type crude oil :-  It has main saturated hydrocarbons Cn H2n+2 (n=1 to 35) i.e.,  CH4  to C35 H72  along with naphthalene and aromatics. Hydro carbons from  C18 H38 to C35 H72  are solids, called waxes. [Saturated hydrocarbons means alkanes;whose G. formula is Cn H2n+2]

- Asphaltic – base type crude oil:-[asphaltic]

It has mainly naphthalene, cyclo paraffin’s with quantity of aromatic and saturated hydrocarbons (alkanes (or) paraffins )

- Mixed base type crude oil:-   This  type of crude oil is a mixture of above two, paraffinic base type and asphaltic type.

Refining of crude oil:-  The crude oils is separated into various useful fractions by fractional distillation and finally converted into  desired specific products the process is called refining of crude oil, and the plants set up for the purpose, are called the oil refineries. The process of refining involves the following steps.

Step 1:-  [Separation of water(cottrell’s process)]

The crude oil from the oil well is an extremely stable emulsion of oil and salt water. The process of freeing oil from water consists in allowing the crude to flow between two highly charged electrodes. The colloidal water is separated from oil in the form of big droplets.

Step 2 :-  [Removal of harmful sulfur compounds].

The crude oil is treated with copper oxide (Cu2O) which gives black precipitate of copper sulfide which can be removed by filtration.

Step 3:-  [fractional distillation]

Crude oil obtained after step 1 and step 2, is then heated at about 4000 c in an iron retort, where by all volatile constituents except the residue (asphalt or petroleum coke) are evaporated. The hot vapors are then passed up a “fractionating column” which is a tall cylindrical tower containing a number of horizontal stainless steel trays, at short distances. Each tray is provided with small chimney, covered with a loose cap. As the vapor go up, they become gradually cooler and fractional condensation takes place at different heights of column. Higher boiling fraction condenses first, while the lower boiling fractions turn – by- turn.

Synthetic petrol:-  (Gasoline)

Petrol is mainly obtained from crude oil but can also be obtained by synthetic process. Two synthetic processes are given here viz;

- Fischer – Tropsch process (ii) Bergius process.

- Fischer – Tropsch process:-

The process was developed by German scientist F.Fischer and H.Tropsch in 1923.

Water gas (CO +H2O) is produced by passing steam over heated coke and is mixed with hydrogen. The gas is purified by passing through Fe2O3 (to remove H2S) and then into a mixture of Fe2O3+Na2CO3 (to remove organic sulfur compounds). The purified gas is compressed to 5-30 atmosphere and then led through a convertor consisting of a mixture of 100 parts cobalt, 5 parts thoria, 8 parts MgO and 200 parts keiselguhr earth, maintained at about 2000-3000c A mixture of saturated and unsaturated hydrocarbons result.

n CO + (2n+1) H2			 CnH2n+2 + nH2O

n CO+2nH2                                           CnH2n +n H2O crude oil

n CO +(2n+1)H2                                                  CnH2n+2 + n H2O

CO+H2O                                      CO2     +     H2

The reaction is exothermic, so out coming hot gaseous mixture is led to a cooler, where a liquid resembling crude oil is obtained. The crude oil thus obtained is then fractionated to yield gasoline and high boiling heavy oil. The heavy oil is reused for cracking to get more gasoline.

All the above reactions take place at 30 atm pressure and 200-3000C temperature. The gasoline formed by this process has loud anti knock value.

- Bergius process :-

This process is oldest one. In this process low ash content coal is taken and powdered. It is mixed with heavy oil to make a paste along with a catalyst (nickel oleateor 5% Fe2O3). This paste is heated with hydrogen at 4500c and under a pressure  200-250 atm for about 1.5 hours, during which hydrogen combines with coal to form saturated hydrocarbons, which decompose at prevailing high temperature and pressure to yield low boiling liquid hydrocarbons. The issuing gases ( from the reaction to vessel) are led to condenser, where a liquid resembling crude oil is obtained, which is then fractioned to get, gasoline, middle oil and heavy oil. The middle oil is hydrogenated to get more gasoline and the heavy oil is used again for making paste. The yield of gasoline is about 60% by this process.

Cracking:-

Cracking is A process of conversion of bigger hydrocarbon molecules into smaller hydrocarbons of lower molecular  weights.

Eg:-           C10H22cracking C5 H12      +    C5 H10

(decane) 	(n- pentane)       (pentene)

Saturated           unsaturated

Hydrocarbon    Hydrocarbon

B.P = 360 C

Of all the fractions obtained by   fractionation of petroleum, gasoline has the largest, demand as a motor fuel, but the yield of these fractions is only 20% of the crude oil. Also the quality of so – called “straight – run” gasoline is not high. It has to be properly blended. Moreover, there is a surplus of heavier petroleum fractions.

To overcome these difficulties, the middle oil and heavy oil are cracked to get petrol. The petrol made by cracking has for better characteristics than “straight-run” petrol.

The process oil cracking is mainly of two types.

- Thermal cracking

- Catalytic cracking

- Thermal cracking:- The heavy oils are subjected to high temperature and pressure, when the bigger hydrocarbon molecules break down to give smaller molecules of the paraffin, olefins plus  some  hydrogen. This process may be carried out either in liquid phase” or vapour – phase”.

- Liquid –phase thermal cracking :-  The heavy oil is cracked at a suitable  temperature of 4750 - 5300c, and under pressure of 100 Kg/cm2. The cracked products are then separated in a fractionating column. The yield is 50-60% and action rating of the petrol produced is 56-70.

(b) Vapour – phase thermal cracking :-  The  process is suitable only for those oils, which may be readily vaporized(low boiling point). It requires less time than the liquid phase method. The cracking oil is first – vaporized and then cracked at about 6000- 6500C under a low pressure of 10-20 Kg/cm2. It requires less time than the liquid phase method. Petrol obtained from vapour – phase cracking has better anti – knock properties, but poorer stability than petrol produced from liquid – phase cracking.

(2) Catalytic cracking: - The quality and yield of gasoline produced by cracking can be greatly improved by using a suitable catalyst like aluminum silicate [Al2(SiO2)3] or aluminum oxide [Al2O]. This process completes at lower temperatures and lower pressure. When compared to the thermal process (3000 – 4500C temp. and 1-5 Kg/cm2 pressure). Catalytic cracking is also of two types.

(a) Fixed – bed catalytic cracking:-

The oil vapour are heated in a pre heater to cracking temperatures (4200-4500c) and then forced through a catalytic chamber (containing artificial clay mixed with zirconium oxide) maintained at 4250 – 4500c and 1.5 Kg/cm2pressure. During their passage through the tower, about 40% of the charge is converted into gasoline and about 2-4% carbon is formed. The latter gets adsorbed on the catalyst bed. The vapors produced are then passed through a fractionating column, where heavy oil fractions condense. The vapors are then led through a cooler, where some of the gases are condensed along with gasoline and uncondensed gases more on. The gasoline containing some dissolved gases is then sent to a ‘stabilizer ‘where the dissolved gases are removed and pure gasoline is obtained.

The catalyst, after 8-10 hours, stops functioning, due to the deposition of black layer of carbon, formed during cracking. This is re-activated by burning of the deposited carbon. During the re- activation, the vapors are directed through another catalyst chamber.

(b) Moving – bed catalytic cracking:-The solid catalyst  is very finely powered, so that it behaves almost as a fluid, which can be circulated in gas stream. The vapors of cracking stock (gas oil, heavy oil, etc.) mixed with fluidized catalytic is forced up into a large reactor ‘bed’  in which cracking of the heavier into lighter molecules occurs. Near the top of the reactor, there is a centrifugal separator (cyclone), which allows only the cracked oil vapors to pass onto the “fractionating column”, but retains all the catalyst powder in the reactor itself. The catalyst powder gradually becomes heavier, due to coating with carbon, and settles to the bottom, from where it is forced by an air blast to regenerator (maintained at 6000c ).

In regenerator, carbon is burnt and the regenerated catalyst then flows through a stand pipe for mixing with fresh batch of incoming cracking oil. At the top of the regenerator, there is a separator which permits only gases (CO2 etc.) to pass out but holds back catalyst particles.

KNOCKING

In internal combustions, a mixture of gasoline vapour and air is used as a fuel. After the initiation of the combustion reaction by spark in the cylinder, the flame should spread rapidly and smoothly through the gaseous mixture, there by the expanding gas drives the piston down the cylinder.

The efficiency of an engine is directly related with compression ratio (CR), “The ratio of the gaseous volume in the cylinder at the end of the suction stroke to the volume at the end of the compression – stroke of the piston, is known as compression ratio”.  More CR value better will be engine. CR depends upon the type of constituents in gasoline”

“In certain circumstances (due to the presence of some constituents in the gasoline used), the ratio of oxidation becomes so great that the last portion of the fuel air mixture gets ignited instantaneously (immediately), producing an explosive violence, known as knocking”. Knocking decreases the efficiency of engine.

The tendency of knocking is based on the chemical structure of hydrocarbons.

Straight- chain paraffins > branched – chain paraffins> olefins > cyclo paraffins > aromatics

In the above order, the knocking of fuel constituents decreases and anti – knocking increases from left to right.

The anti –knocking value of fuel can be increases by adding tetra – ethyl lead (TEL) --- [(C2H5)4 Pb ].

Octane number (or) Octane Rating:-Knocking of fuel (gasoline), can be measured in terms of octane  number (or)  octane rating, it has been found that n-heptane knocks very  badly hence , its anti – knock value has arbitrarily been given “zero” on the other hand, isooctane (2,2,4- trimmethyl pentane) gives very little knocking so its anti – knock value has been given as “100”.

“Octane number of a gasoline (or any other internal combustion engine fuel) is the percentage of isooctane in a mixture of isooctane and n- heptane, which matches the fuel under test in knocking characteristics”. In this way, an ’80- octane” fuel is one which has the same combustion characteristics as a 80:20 mixtures of isooctane & n- heptane.

n- heptanes

cetane number

diesel knock can be defined as the ratting sound produced due to ignition –lag i.e. the delay in the ignition of the fuel

it can be defined as the percentage of hexadecane in a mixture of hexadecane and 2- methyl nephthalene whose diesel knock matches with the diesel knock of the test sample under examination

n-hexa decane                 2- methyl naphthalene

the order is n-alkanes> naphthalenes > alkenes >. Branched alkanes > aromatics

Gaseous Fuels:-  Natural gas is the primary  gaseous fuel. A variety of secondary fuel are obtained from coal or petroleum’s. They include coal gas, produces gas, water gas derived from coal. LPG 
(liquid petroleum gas) and CNG (compressed natural gas) are derived from natural gas and oil gas is formed by cracking of kerosene oil [ producer gas  -- mixture of CO+ N2 water gas--- mixture of CO+H2O].

Natural gas:-  Natural gas is obtained from oil wells dug in the oil bearing regions. When natural gas occurs along with lower hydrocarbon   like CH4,C2H6, it is called “wet gas” the wet gas is treated to remove propane, propene, butane, butene, which are used as LPG  [ CH4 – methane, C2H6 ]

On the other hand when the gas is associated with higher hydrocarbon along with CH4, is called “dry gas”. Before use, the natural gas is purified to remove objectionable ingredients  such as water, dust grit, H2S, CO2, N2 and heavier liquefiable hydrocarbons (propane, propene, butane, butene, etc.).

The approximate composition of natural gas is :

CH4 = 70-90%; C2H6= 5-10%; H2 = 3%; CO+CO2 = rest

The calorific value varies from 12.000 – 14,000 kcal/m3

Uses of Natural gas:-

- It is excellent domestic fuel and can be conveyed over very large distances in pipelines.

- It has recently been used in the manufacture of a number of chemicals by synthetic processes.

- It is also used as raw material for the manufacture of carbon black and hydrogen.

- A synthetic protein (used as animal feed) is obtained by microbiological fermentation of methane (which is main constituent of Natural gas).

Compressed natural gas:- ( CNG)

CNG is natural gas compressed to a high pressure of about 1,000 atmospheres. A steel cylinder containing 15kg of CNG contains about 2 104L or 20 m3 of natural gas at 1 atmospheric pressure. CNG is now being used as a substitute for petrol and diesel, since it is comparatively much less pollution causing fuel. During its combustion, no sulphur and nitrogen gases are evolved. Moreover, no carbon particles are ejected. Hence it is a better fuel then petrol or dieset for automobiles. However initial cost of engine designed to use CNG as a fuel is comparatively higher then that of engine designed to use petrol or diesel. In Delhi is is mandatory for all buses, taxis and auto to use CNG as fuel.

Advantages of CNG:-

- Due to higher temperature of ignition, CNG is better fuel than petrol and diesels and hence it is a much safer fuel.

- Operating cost of CNG is less.

- It releases least pollutants like CO and unburnt hydrocarbons.

- The conversion of gasoline operated automobiles into CNG operated vehicle is very easy.

- It  underges regular combustion.

- No. anticnocking agent is requird as it has high octane number.

Liquid petroleum gas:- (LPG) [butylenes/ butane/propylene/ propene]

LPG is obtained from natural gas a byproduct in refineries during cracking of heavy petroleum products. The main constituents of LPG are n-butane, isobutene, butylenes and propane, with little or no propylene and ethane. LPG is supplied under pressure in containers under the trade name like Indane, Bharat gas, HP gas ,etc.

Advantage of LPG:-

- It has high calorific value 27800 kcal/m3. The calorific value of LPG is roughly 3 times higher than that of natural gas and 7 times that of coal gas.

- It gives less CO and least unburnt hydrocarbon. So it cause least pollution.

- It gives moderate heat which is very good for cooking.

- Its strong, simple and is colourless.

- It has tendency to mix with air easily.

- It on burning gives no toxic gases though it is highly toxic.

- It neither gives smoke  nor ash content.

- It is cheaper than gasoline and hence used as fuel into auto vehicles.

- Needs little   for maintenance purpose.

Biogas : Biogas  is produced by the degradation of biological matter by the bacterial action (of anaerobic  bacteria ) in the absence of free oxygen.Ex:- Natural gas is a biogas, which results after a long period decay of animal and vegetable matters, buried inside the earth , gobar gas (or dung gas), which is produced by the anaerobic fermentation of cattle dung. Biogas can also be produced from the sewage waste, and other organic wastes.

Constituents:- CH4 (Methane ) – 50 -60%(combustible gas)

CO2 – 30-40% (a non combustible gas)

H2 – 5-10% (a combustible gas)

N2 –2-6% (a non combustible gas)

H2S – traces (a  combustible gas)

Out of these, the constituent methane (an extremely good fuel) makes biogas an excellent fuel.

Raw materials for biogas :- Animal  dung, poultry wastes, vegetable wastes, waste paper, cotton cloths, plants wastes, human excreta , bird’s excreta, etc.

Advantage of biogas:- (1) Biogas production is very economical. It has been found that 1kg of dry cattle dung gives about 160 liters of gobar  gas, which can supply 800 kg of heat. On the other hand , 1 kg of dry dung on direct burning gives only 100kg of effective heat.

(II) The gas has all advantages of gaseous fuel like cleanliness, absence of smoke, flexibility, etc.

(III) It does not contain poisonous gas CO, as an ingredient.

(IV) It provides  simultaneously excellent yield of good manure.

Limitation of biogas:-  It is necessary to have the gas lamp or store or burner with in 10 meters of the plant.

Combustion:-Combustion is an exothermic chemical reaction, which is accompanied by development of heat and light at a rapid rate ,so, that temperature rises considerably. For example, combustion of carbon in O2.

C (S) + O2 (g)                    CO2 (g) + 97 Kcal

For proper combustion, the substance must be brought to its kindling (or) ignition temperature; which may be defined as the minimum temperature at which the substance ignites and burns without further addition of heat from outside.

Analysis of flue gas:- Gases coming out after combustion ( CO,CO2, O2) are called fuel gases .Analysis  of flue gases gives idea about complete or incomplete combustion process. In case of incomplete combustion, the concentration of CO will be more (i.e., not complete conversion of carbon of fuel to CO2).it shows less supply of oxygen for combustion process. On the other hand, if the flue gases contain a considerable amount of oxygen, it indicates the oxygen supply is in excess, though the combustion may be complete.

The analysis of flue gases is carried out with the help of Orsat’s apparatus. It consists of a horizontal tube. At one end of this tube, there is a three way stop- cock and the other end is connected with a burette. The burette is graduated and surrounded by a water jacketed to keep the temperature of the gas be to constant during the experiment. The burette is connected in series to a set of three absorption bulbs, each through a separate stop cock. The lower end of the jacketed burette is further connected to a water reservoir with the help of rubber tube. The water level in the burette can be raised or lowered by raising or lowing the water reservoir containing water. The other end of horizontal tube which is connected to three way stop cock is further connected to a U- tube. The U- tube is packed with fused CaCl2 and glass wool for drying fuel gas and for avoiding the incoming  of any smoke particles respectively .

The three absorption bulbs apart from having solution for absorbing CO2, O2 and CO also  filled with glass tubes, so that surface area of contacts between the gas and the solution is increased. The first bulb has potassium hydroxide (KOH) solution and it absorbs only CO2. The second bulb has alkaline pyrogallic acid and it can absorb O2 and CO2. The  third bulb contains ammonical cuprous chloride and it can absorb CO,O2 and CO2. For proper analysis, it is necessary that the flue gas is passed first through a bulb containing KOH where CO2 is absorbed. Then it is passed through second bulb containing alkaline Pyrogllic acid, where only O2 will be absorbed. Although it can also absorb CO2 but all O2 has already been removed by KOH. finally, flue gases are passed through third bulb containing ammonical cuprous chloride, where only CO will be absorbed.

Working:-  The whole apparatus is thoroughly cleaned, stoppers greased and then tested for  air- tightness. The absorption bulbs are filled with their respective solution. Then their stop cocks are closed. The water reservoir and jacket and filled with water. Air is first excluded from the burette by raising the water reservoir till the burrette is completely filled with water. For the exclusion of air, the three way stop cock should be opened to the atmosphere. Next, the flue gas to be analyzed is drawn in by lowering the water reservoir and connecting the three way stop cock to flue gas supply. For better result, air from the capillary connecting tubs should be expelled by repeating the above process of sucking and expelling the flue  gas by lowering and raising the water reservoir.

Finally , 100ml of fuel gas is carefully sucked in the burette for analysis. The three way stop cock is  then closed. The flue gas is forced in the bulb 1 by opening its stop cock and raising the water reservoir. Bulb 1 contains KOH solution wich absorbs CO2. For the complete absorption of CO2, the flue gas is sent, 2 or 3 times again and again in bulb 1.

The unabsorbed gas is finally taken back in the burette and  then stop cock for CO2absorption bulb is closed. The levels of water in the reservoir and burette are equalized and the volume of residual gas is noted. The decrease in volume  gives the volume of CO2 in 100ml of the flue gas sample. The volume of CO2 and CO are similarly determined by passing the flue gas through absorption bulbs 2 and 3 respectively. The gas remaining in burette after absorption of CO2,O2 and CO is taken as Nitrogen. The percentage of CO in the flue gas should be measured quite carefully since it is present to very small extent in the flue gas.

By knowing the weights of the gases present and their molecules the percentage by volume can be obtained.  Suppose the gases g1, g2, g3 are present as a1gm, a2gm, a3gm and have their molecular weights as m1, m2, m3 respectively.  Then their proportional volumes are , , and the total volume as .  Let the total volume be denoted as M, then the

Percentage of gas g1 =

Percentage of gas g2 =

Percentage of gas g3 =

UNIT-V:

ENGINEERING MATERIALS AND CORROSION

POLYMERIC MATERIALS:

Classification of polymers - Thermoplastics & Thermosetting resins.Types of Polymerization (i) Addition (ii) Condensation (iii) Co-Polymerization. Natural and synthetic Elastomers.

Conducting polymers : Introduction, Classification and Mechanism of conduction in Poly-acetylene,Applications of conducting polymers.

Biodegradable polymers: Introduction preparation, properties and applications of polylactic acid

Polymeric Materials:

Polymer is a macro molecule formed by the union of many small molecules.

Depending upon the structure, a polymer may be linear or branched.

| Ex. – CH2 – CH2 – | CH2--Si – O – Si – O - Si |
| --- | --- |
| Polyethylene | ¦ |
| Linear | CH2- Si – O – Si – O – |
|  | Branched silicon resin. |

Depending type of monomer, they may be homopolymer (made up of same monomer) or

Co-polymer (made up of different monomers)

Classification of polymers

THERMOSET PLASTICS:They are formed by condensation polymerization. Theyhave three

dimensional network structures. The cross links and bonds retain their strength on heating and

hence they do not soften on heating. On prolong heating however, charming of polymers is

caused and more brittle. They cannot be reclaimed from wastes. Due to	 strong bonds and cross-links they are insoluble in a. They retain the shape and structure even on heating. Hence, they cannot be reshaped andreused.They are usually, hard, strong lmost all organic solvents.

Difference between thermo set and thermoplastics.

Plastics are materials that show the property of plasticity and can be moulded into any desired	 shape and dimension of articles by the application of heat and pressure

| Thermoplastics | Thermosets Plastics |
| --- | --- |
| - These resins become soft on heating and rigid on cooling reversibly - The heating and cooling donot alter the chemical nature of these resins because the changes involved are purely of physical nature. - These are formed by chain polymerization - They consist of long chain linear polymers with weak secondary – vander waal’s forces of attraction between them. - They are softened on heating readily because the secondary force of attraction between the individual chain can break by heat, pressure or both - They can be reshaped - These are soft, weak and less brittle - These resins are usually soluble in organic solvents - Polyethylene, polyvinyl chloride are the examples. | 1.During fabrication process these resins are moulded once they are solidified they cannot be softened 2.These resins are permanent setting resins 3.These are formed by step polymerization 4.They have three dimensional network structure 5.The bonds retain their strength on heating hence they do not soften again on heating. 6.Cannot be reshaped 7.These are hard, strong and more brittle 8.Due to strong bonds and cross links, they are insoluble in almost all organic solvents. 9.Bakelite, Nylons, etc, are the examples. |

POLYMERISATION: It may be defined as the process of linking or joining together small molecules like monomers to make large molecules.		Basically there are 3 types of polymerizations.

1. Additional polymerization or Chain polymerization

2.Condensation polymerization or Step polymerization

3.Copolymerization

- Additional polymerization or Chain polymerization:This polymerization yields an

exact multiple of basic monomeric molecules. This monomeric molecule contains one or more double bonds. By intermolecular rearrangement of these double bonds makes the molecule bifunctional. In this polymerization process light, heat and pressure or catalyst is used to breakdown the double covalent bonds of monomers.

- Condensation polymerization or Step polymerization:May be defined as “a reaction

occurring between simple polar-group-containing monomers with the formation of polymer and elimination of small molecules like water, HCl, etc.” For example, hexamethylene diamine and adipic acid condense to form a polymer, Nylon6:6. Additional polymerization is a chain reaction converting of a sequence of three steps.

Initiation, propagation and termination.

- Initiation step is considered to involve two reactions. The first is the production of free radicals, usually, by the hemolytic dissociation of an initiator (or catalyst) to yield a pair of radicals R’.

I	2R’	………(1)

(Initiator)	(Free radicals)

The second part of initiation under the addition of this radical to the just moment molecule (M) to		 produce the chain initiating species M1.

the polymerization of monomer CH2 = CHY taken in the    form.

- Propagation step: Consists of the growth of M1 by successive additions of large numbers of

monomer molecules according to equation

| M1+ M M2 |  |
| --- | --- |
| M2+ M M3 |  |
| M3+ M M4 or in general terms | Mn + M Mn + 1 |

- Termination step: At some time, the propagation polymer chain steps growing and terminates.

Disproportion in which a hydrogen atom of one radical center is transferred to another

radical center. This results in the formations of two polymer molecules, are saturated and one

unsaturated

e.g.

The two different modes of terminations can be represented in general terms by:

M*n + M*m   M*n+m	(Coupling)

M*n + Mm*Mn + Mm   (Disproportionation)

3.Co-polymerization: Polymerization involving two different monomers.

Ex. Polymerization of butadiene and styrene to gave Buna –S.

Natural Rubber:Rubbers also known as Elastomers, they are high polymers, whichhave elastic

properties in excess of 300%.

Natural rubbers consist of basic material latex, which is a dispersion of isoprene. During the

treatment, these isoprene molecules polymerize to form, long-coiled chains of cis-polyisoprene.

Natural rubber is made from the saps of a wide range of plants like Hevea brasillians and guayule		Latex: is a milky white fluid that oozes out from the plant Hevea brasillians when a cut is made onthe steam of the plant.		The latex is diluted with water. Then acetic or formic acid is added [1kg of acid per 200kgs of latex] 	to prepare coagulum. This is processed to give wither crepe rubber or smoked rubber

Vulcanization:		Vulcanization discovered by Charles Goodyear in 1839.		It consists of heating the raw rubber at 100 – 1400C with sulphur. The combine chemically at the 		double bonds of different rubber spring and provides cross-linking between the chains. This cross-	linking during vulcanization brings about a stiffening of the rubber by anchoring and consequently	preventing intermolecular movement of rubber springs. The amount of sulphur added determines the 	extent of stiffness of vulcanized rubber. For example, ordinary rubber (say for battery case) may contain as much as 30% sulphur.

Advantages of vulcanization:

- The tensile strength increase.

- Vulcanized rubber has excellent resilience.

- It has boarder useful temperature range (-40 to 1000C)

- It has better resistance to moisture, oxidation and abrasion.

- It is resistance to organic solvents like CCl4, Benzene petrol etc.

- It has only slight thickness.

- It has low elasticity

CONDUCTING POLYMERS :

Most polymeric materials are poor conductor of electricity, because of the non – availability of large number of free electrons in the conduction process.

“ The polymeric materials, which possess electrical conductivities on par with metallic conductors, are called conducting polymers”

Conductivities as high as 1.510-1 ohm-1 m-1 have been attained in these polymeric materials. On a volume basis, this value is equal to one fourth of the conductivity of copper (or) is twice its conductivity on the basis of weight. Electrical conductivity of some polymers are given below.

|  | Polymers | Electrical conductivity (ohm-1 m-1 |
| --- | --- | --- |
| 1 | Phenol formaldehyde | 10-9 – 10-10 |
| 2 | Poly – methyl methacrylate | < 10 -12 |
| 3 | Nylon – 6,6 | 10-12 – 10-13 |
| 4 | Polystyrene | < 10-14 |
| 5 | Polyethylene | 10-15-10-17 |
| 6 | Polytetrafluoro ethylene | <10-17 |

Different types of conducting polymers are discussed below:

- Intrinsically conducting polymer (ICP) (or) Conjugated – Electrons conducting polymer

- Doped conducting polymer

- Extrinsically conducting polymer

- Coordination conducting polymer (inorganic polymer)

(1) Intrinsically conducting polymer (ICP) or Conjugated – Electrons conducting polymer

The polymers which conduct electricity are called conducting polymers. The conduction of the polymers is may be due to the presence of double bonds in the structure (Intrinsic Conducting Polymers) or due to the presence of externally added ingredients in them (Extrinsic Conducting Polymers).

Intrinsic conducting polymers are classified into two types.

- Conducting polymers having conjugation:

These polymers have conjugated double bonds in the backbone and possess their conductivity due to pi-electrons.The orbitals of conjugated electrons overlap the entire backbone of the polymer and result in the formation of valence bands and conduction bands, which were separated by a significant Fermi energy gap.The electrical conductivity will takes place only after thermal or activation of the electrons, which give them sufficient energy to jump the gap and reach into conduction band.

Ex:

Polymerization of acetylene over Zeigler Natta catalysts will give polyacetylene, which is predominantly in cis-form. On rising the temperature of the film gives more stable trans-form.

- Doped conducting polymers:

The conducting polymers having pi-electrons in their backbone can be easily oxidized or reduced because of their low ionization potential and high electron affinities. Hence theirconductance can be increased by the introduction of a +ve charge or –ve charge on backbone by oxidation or reduction.

If the conductivity increased by the oxidation process by adding some alkali metal ion or electron acceptor, then it is said to be “p-doping”. If the conductivity is increased by reduction process by adding an electron donor is called “n-doping”.

- Oxidation process  (p-doping):

In this process, -electrons are removed from conjugated double bonds and positive hole or radical cation is created. The radical cation is called ‘polaron’ whichstabilizes itself by polarizing the medium.  The polarons are mobileand this delocalization is responsible for the conduction of current in the polymers. The p-doping is generally brought by adding Lewis acids (such as FeCl3), iodine, bromine arsenic pentafluoride (AsF5) etc.

Ex: polyacetylene

Doping of polyacetylene:

Reduction process (n-doping):

In this type of doping some electrons are introduced into the polymer by reducing agent like sodium naphthalide[Na+(C10H8)-]or by adding Lewis base (electron donor).  Formation of polaron, bipolaron takes place in two steps, followed by recombination of radicals, which yieldstwo charge carriers on the polyacetylene chain, which are responsible for the conduction as shown below.

The n-doping in less common, because on earth, atmosphere is oxygen rich, which creates an oxidizing environment.  An electron rich ‘n-doped’ polymer will react immediately with oxygen and converts into the neutral state by re-oxidizing.

Polyaniline

It can be prepared by the reaction of aniline with ammonium persulfate in aq. HCl.

Poly aniline is a conjugated polymer and in reactive. Polyaniline is considered as an organic metal.Poly aniline is transparent in thin layers. In conducting state it will be in green colour.  It turns red under reducing conditions and blue under oxidizing or basic conditions. Poly aniline is a stable conducting polymer. It has wide range of conductivity (10-11–105 S/cm). It can be synthesized chemically (or) electrochemically as a blue powder or film.

Emeraldine salt is the most stable form of polyaniline. It is also known as “Synthetic metal”, because it possess metallic conductivity, metallic lusture and metallic sound.

-

(3) Extrinsically conducting polymers :

Extrinsically conducting polymers are those polymers whose conductivity is due to the presence of “externally” added ingradient in them. These are of the following two types.

(a) Conductive element filled polymer : It is a resin or polymer filled with conducting elements such as carbon black, metallic fibres, metal acids, etc. In this, the polymer acts as the binder to hold the conducting elements together in the solid entity.

(b) Blended conducting polymer : It is the product obtained by blending a conventional polymer with a conducting polymer either by physical or chemically change.

(4) Coordination conducting polymer :  (inorganic Polymer)

It is a charge transfer complex containing polymer obtained by combining a metal atom with a polydentate ligand.

Application : Conducting polymers are finding increased use because they are light weight, easy to process and have good mechanical properties. Some of the important applications of conducting polymers are :

(A) In rechargeable light weight batteries based on perchlorate doped polyacetylene – lithium system. These are about 10 times, lighter than conventional lead storage batteries. Such batteries are sufficiently flexible to fit a variety of designed configuration.

(b) In optically display devices based on polythiophene. When the structure is electrically based, the optical density of the film changes, i.e., colour changes. Such electrochromic systems produce coloured displays with faster switching time and better viewing than conventional liquid crystal display devices. (LCD)

(C) in wiring in aircrafts and aerospace components

(D) in telecommunication systems

(F) in antistatic coatings for clothing.

Biodegradable polymers

Poly lactic acid

Properties PLA polymers range from amorphous glassy polymer to semi-crystalline and highly crystalline polymer with a glass transition 60–65 °C, a melting temperature 130-180 °C, and a tensile modulus 2.7–16 GPa. Heat-resistant PLA can withstand temperatures of 110 °C.

Applications:

PLA can also be used as a decomposable packaging material

Cups and bags have been made from this material.

In the form of a film, it shrinks upon heating, allowing it to be used in shrink tunnels.

It is useful for producing loose-fill packaging, compost bags, food packaging, and disposable tableware. In the form of fibers and nonwoven fabrics,

PLA also has many potential uses, for example as upholstery, disposable garments, awnings, feminine hygiene products, and diapers.

Due to its bio-compatibility and biodegradability, PLA has also found ample interest as a polymeric scaffold for drug delivery purposes.

Corrosion

Metals and alloys are used as fabrication or construction materials in engineering.  If the metals or alloy structures are not properly maintained, they deteriorate slowly by the action of atmospheric gases, moisture and other chemicals.  This phenomenon of destruction of metals and alloys is known as corrosion.

Corrosion of metals is defined as the spontaneous destruction of metals in the course of their chemical, electrochemical or biochemical interactions with the environment.  Thus, it is exactly the reverse of extraction of metals from ores.

Example:   Rusting of iron a layer of reddish scale and powder of oxide (Fe3O4) is formed on the surface of iron metal.    A green film of basic carbonate [CuCO3 + Cu(OH)2] is formed on the surface of copper, when it is exposed to moist-air containing carbon dioxide. Corrosion is depends of conditions for example gold has excellent resistance to corrosion under atmospheric conditions but it gets readily corroded when exposed to mercury. On the other hand iron readily gets corrode in the atmosphere, but it does not corrode in mercury.

CAUSES OF CORROSION

In nature, metals occur in two different forms.   1)  Native State  (2) Combined State

Native State: Native state means free or uncombined state.  These metals are non-reactive in nature.  They are noble metals which have very good corrosion resistance.  Example:  Au, Pt, Ag, etc.,

Combined State:  Except noble metals, all other metals are highly reactive in nature which undergoes reaction with their environment to form stable compounds called ores and minerals.  This is the combined state of metals.  Example:  Fe2O3, ZnO, PbS, CaCO3, etc.,

Metallic Corrosion:  The metals are extracted from their metallic compounds (ores).  During the extraction, ores are reduced to their metallic states by applying energy in the form of various processes.  In the pure metallic state, the metals are unstable as they are considered in excited state (higher energy state).  Therefore as soon as the metals are extracted from their ores, the reverse process begins and form metallic compounds, which are thermodynamically, stable (lower energy state).  Hence, when metals are used in various forms, they are exposed to environment, the exposed metal surface begin to decay (conversion to more stable compound).  This is the basic reason for metallic corrosion.

Corrosion-Oxidation

Metal            Metallic Compound  +  Energy

Metallurgy-Reduction

Although corroded metal is thermodynamically more stable than pure metal but due to corrosion, useful properties of a metal like malleability, ductility, hardness, luster and electrical conductivity are lost.

CONSEQUENCES (EFFECTS) OF CORROSION

The economic and social consequences of corrosion include

i)  Due to formation of corrosion product over the machinery, the efficiency of the machine gets failure leads to plant shut down.

ii) The products contamination or loss of products due to corrosion.

iii) The corroded equipment must be replaced

iv) Preventive maintenance like metallic coating or organic coating is required.

v)  Corrosion releases the toxic products.

Vi) The cost of corrosion can be expensive. It can impact the health of employees and those who frequent your commercial building.

Vii) Studies have shown that consuming water with high levels of toxic metals – lead and copper – can lead to acute and chronic health problems.

THEORIES OF CORROSION: -

Corrosion is explained by using two types of theories.

(1)CHEMICAL CORROSION; - Direct chemical attack by dry gases on a metal does not take place more corrosion at atmospheric temperature. Whenever corrosion takes place by direct chemical attack a solid film of the corrosion product is usually formed on the surface of the metal which protects the metal from further corrosion. This phenomenon is called passivity.

Ex. AgCl film formed by attack of Cl2

There are mainly three types of chemical corrosion.

(a) Oxidation corrosion

(b) Corrosion by gases like SO2, Cl2, H2S etc.

(c) Action of liquid metals.

(a) OXIDATION CORROSION; - It takes place by direct action of oxygen. Normally in the absence of moisture. At ordinary temperature alkali and alkaline earth metals are attached by O2, while other metals require more temperature.

M --------------> Mn+ +ne-

O2 + 4e-  ------------->	2O2–

------------------------------------------------------------------------------

M	 +  	O2     ---------> MO2      (or)

Metal 	+ 	oxygen   ------> metal oxide.

Mechanism: At the surface of metal oxidation occurs and the resulting metal oxide scale forms a barrier which restricts further oxidation. For oxidation to continue either the metal must diffused outwards through the scale to the surface or the oxygen must defuse inwards through the scale to the underlying metal. Both the cases are possible. But the outward diffusion of metal is generally more rapid than inward diffusion of oxygen since metal ion is appreciably smaller than the oxygen ion and hence more mobile.

Metal + Oxygen ----------------> Metal oxide

When oxidation starts, a thin layer of oxide is formed on the metal surface and the nature of this film decides further action. If the film is,

i. Stable layer:   A stable layer is fine grained in structure and can get adhered tightly to the metal surface. Such film behaves as protective coating in nature, thereby shielding the metal surface.  The oxide films on Al, Sn, Pb, Cu, Pt, etc.,

ii. Unstable oxide layer:  An unstable oxide formed on the metal surface decomposes back into the metal and oxygen. Hence, oxidation corrosion is not possible with noble metals.

Ex. Ag, Au, Pt.

iii. Volatile oxide layer: If corrosion product is volatile it evaporates as soon as it is formed.  Leaving the metal for further attack..  This causes rapid and continuous corrosion.

Ex: MoO3.

iv. Porous layer:   The layer having pores or cracks.  In such a case, the atmospheric oxygen has access to the underlying surface of metal, through the pores or cracks of the layer, corrosion continues, till the entire metal is completely converted into its oxide.

Ex: alkaline and alkaline earth metal oxides

(b) CORROSION BY OTHER GASES: - the gases like SO2, H2S CO2, Cl2 etc. also causes chemical corrosion andtThe extent of corrosion depends on the chemical affinity between the metal and gas involved in the reaction. The corrosion product is metal compound. The extent of corrosion is depends on nature of metal compounds. If the film formed is nonporous or protective then corrosion is limited to only external surface.

Ex. AgCl film formed by attack of Cl2 on Ag metal.

If the film formed is porous or volatile then corrosion takes place more extent.

Ex. SnCl2 formed by attack of Cl2 on Sn metal.

(c) ACTION OF LIQUID METALS; The chemical action of a flowing liquid metal at high temperature, on a solid metal or alloy produces liquid metal corrosion. The reason for corrosion is the dissolution of the solid metal by liquid metal or internal penetration of the liquid metal into the solid phase.

(2) ELECTROCHEMICAL CORROSION (WET CORROSION); - The corrosion of metals in aqueous environment is more than that of dry conditions. According to electrochemical corrosion it is a two step process which occurs simultaneously oxidation and reduction. If a piece of metal is dipped in water or a solution it has a tendency to goes into solution. Some of its atoms may lose electrons and goes into solution as positively charged ions. For the progress of corrosion, anodic and cathodic areas must exist in electrical contact with each other. It is explain by using two types of principles.

(a) Galvanic cell principle.     (b) Concentration cell principle.

(a)GALVANIC CELL PRINCIPLE: - A galvanic cell contains two different electrodes dipped into its own ionic solutions and electrically connected to each other.

Ex; - Zn and Cu rods dipping in solution having zinc and copper ion solutions. The solutions are separated by a salt bridge. The emf developed is due to the reaction taking place at electrodes.

Zn   ---------------------------->Zn2+ + 2e–    	at anode

Cu2+ + 2e–   ---------------------->Cu               	at cathode

Thus zinc dissolved and copper deposited.

The concept of corrosion reaction can be developed on the basis of reaction occurring in a Daniel cell. The greater the potentials difference between the two metals in actual contact the faster will be the corrosion. The electrode potential is a measure of the tendency of a metal to go into solution and depending upon several factors like nature of metal, concentration of solution and temperature.

In general for any metal containing negative potential and being in contact with a conducting solution of moisture, tiny galvanic cell is formed. The type of chemical reaction may be different in different cases. At anode there will be always dissolution of metal and at the cathode may be reduction of H3O+ or O2 takes place

(b) CONCENTRATION CELL PRINCIPLE: - from the above discussion that for a metal to corrode it must be in contact to some other metal which is cathodic to it, and that a pure metal should not corrode at ll. But the electrochemical theory also provides an explanation for the corrosion of a single metal through the formation of electrolyte concentration cell. When a metal is dipped in a salt solution there will be the tendency for the metal around which concentration is lower, to go into solution, and metal will be deposited at other plate. The surface exposed to a solution of lower concentration looses electrons hence becomes anode and dissolves, and surface exposed to a solution of higher concentration becomes cathode as it receives electrons.

Thus an emf will be produced. This shows that if a metal piece is in contact with a solution whose concentration is different at different ends of the piece, a concentration cell will be set up and corrosion will take place where the concentration of the electrolyte is lower. Such concentration difference of an electrolyte can be caused by differential aeration at the electrodes. One is well aerated and another is less aerated so the less aerated has less concentration of O2 hence it acts as anode.

Mechanism:

Electrochemical corrosion involves flow of electrons between anode and cathode. The anodic reaction involves dissolution of metal liberating free electrons.

M→ Mn+ + ne-

The cathodic reaction consumes electrons with either evolution of hydrogen or absorption of oxygen which depends on the nature of corrosive environment.

Evolution of hydrogen: This type of corrosion occurs in acidic medium e.g. considering the metal Fe, anodic reaction is dissolution of iron as ferrous ions with liberation of electrons.

Fe → Fe2+ + 2e- ( Oxidation)

The electrons released flow through the metal from anode to cathode, whereas H+ions of acidic solutionare eliminated as hydrogen gas.

2H++2e-→  H2

The overall reaction is

Fe+ H+→ Fe2+ + H2

Absorption of oxygen: - For example, rusting of iron in neutral aqueous solution of electrolytes in presence of atmospheric oxygen. Usually the surface of iron is coated with a thin film of iron oxide. If the film develops cracks, anodic areas are created on the surface. While the metal parts act as cathodes. It shows that anodes are small areas, while the rest metallic part forms large cathodes.

At anode:

Fe→  Fe2+ + 2e- (Oxidation)

At cathode:

The released electrons flow from anode to cathode through iron metal.

½ O2 + H2O + 2e- → 2OH- (Reduction)

Fe2+ + 2OH-→  Fe(OH)2

(a) If oxygen is in excess, ferrous hydroxide is easily oxidized to ferric hydroxide.

4Fe (OH)2 + O2 + 2H2O→  4Fe (OH)3

The product called yellow rust corresponds to Fe2O3. xH2O.

TYPES OF CORROSIONS

(I) Galvanic corrosion.                (II) Pitting corrosion.

(V) Water line corrosion.

(I) GALVANIC CORROSION: -

When two dissimilar metals are electrically connected and exposed to an electrolyte, the metal higher in electrochemical series undergoes corrosion. This type of corrosion is called Galvanic corrosion.

e.g., Zinc (higher in electrochemical series) forms the anode and is attacked and gets dissolved; whereas copper (lower in electrochemical series) acts as cathode.

Zn →Zn2+ + 2e- (Oxidation)

Cu2+ + 2e–   ---------------------->Cu  (reduction)

(ii) PITTING CORROSION: - when once corrosion starts on a metal surface at certain spots they become less open to oxygen with the results that corrosion proceeds faster at those spots making deeper and deeper activities. This is known as pitting. Pitting is easy to explain on the basis of differential aeration. The surface under the rust or scale has less oxygen, there for is anodic and corrodes. As corrosion proceeds, a cavity is created and as at becomes deeper its bottom becomes still less oxygen which makes it still more anodic.

(V) WATER LINE CORROSION:- A drop of water on an iron or steel plate is an interesting illustration for the study of the formation of an oxygen concentration cell. The concentration of oxygen at the center of the drop is less than its surface. Thus water drop centre becomes anodic and the surface region becomes cathodic.  Corrosion occurs at the anodic part the metal just below the water level.

Fe  ------------->   Fe+2 + 2e–

4H++ O2 + 4e– -----------> 2H2O (2H+  + 2OH–)

Fe+2 + 2OH–     ------------->   Fe (OH)2

Fe (OH)2 is gradually changed to Fe (OH)3 and it is precipitated as rust.

The reaction taking place in the drop can be observed by adding ferroxyl indicator on placing a drop of this dilution on a polished iron surface a concentration cell is formed. The centre becomes anode, and the solution in the centre turns blue because of ferrous ions. Cathode is formed at the edge of the drop and the solution there turns pink because of OH–.

It is quite common in ships, iron tanks used for storage of water.

FACTORS EFFECTING CORROSION ON PURE METAL:The rate and extent of corrosion of pure metal mainly depends on

(a) Nature of metal and (b) nature of the environment.

NATURE OF METAL: - The alkali metals are highly corrosive on account of their tendency to lose electrons and attain a noble gas configuration. The corrosion of metals decreases as we go from left to right across the periodic table and increased form top to bottom.

(a)GALVANIC SERIES: - It is used to provide sufficient information in predicting the corrosion behavior in a particular set of environmental conditions. Oxidation potential measurement of various metals and alloys has been made using standard calomel electrode as the reference electrode and immersing the metals and alloys in sea water. These are arranged in decreasing order of activity and this series is known as the galvanic series.

(b) OVER VOLTAGE: - when a metal which is at a high position in galvanic series is placed in a dilute acid it undergoes corrosion with deposition of a film on its surface and evolution hydrogen gas. So the initial rate of corrosion is high ( due to over voltage) but decreases after a while due to salt film and H2 film surrounding the metal. So an over voltage reduces the corrosion rate. Presence of ions like Cu+2 in the present example lead to deposition of cu metal acting as minute cathodes and hence reduction of over voltage and increase in corrosion.

(c) PURITY OF METAL: - Impurity in metals causes heterogeneity and hence small localized galvanic cells and therefore an enhancement of corrosion. For example if zinc metal is contaminated with impurities like Pb or Sn which are at higher level than Zn, they use zinc as sacrificial anodes and hence more corrosion is seen in comparison to pure metal.

(d) NATURE OF OXIDE FILM: -The metal oxide is protective when the volume of the oxide is greater than the volume of the metal and the metal oxide is non protective when the volume of the oxide is less than the volume of the metal from which it is formed. This is called pilling-bedworth rule.

(e) NATURE OF CORROSION PRODUCT: - If the corrosion product is insoluble in the medium or interacts with the medium to form another insoluble product, then the corrosion product acts as a physical barrier and prevents further corrosion. Similarly the volatile corrosion products increase the corrosion rate. For example molybdenum oxide is volatile and results in excessive corrosion.

INFLUENCE OF ENVIRONMENTAL CONDITION ON CORROSION: -

a. CATHODIC AND ANODIC AREAS: - rate of corrosion is also depends on relative proportion of anodic and cathodic areas. If there is a large cathodic area with only a small anodic area, more corrosion takes place at anode area.

Rate of corrosion α cathodic area / anodic area

b. EFFECT OF PH: In the corrosion reaction H+ or OH- are also involved. Therefore the effect of pH is obvious. As a general rule, acids are more corrosive than neutral or alkaline solutions. Exceptions to the rule are amphoteric metals, aluminium, zinc and lead, which form anions as well as cations. In acid solutions these react quickly like other metals but in alkaline solutions they form complex ions and go into solution. Consequently these metals most resistant to corrosion in neutral solutions. In the case of other metals the corrosion rate is higher in acidic solutions than in alkaline solutions.

c. EFFECT OF TEMPERATURE: - the effect of temperature on the corrosion rate is complicated. The rate of chemical reaction increases with rise in temperature. In this case corrosion is also increase with rise in temperature. But the solubility of gases decreases with increase in temperature. So corrosion decrease with increase in temperature. The rate of corrosion due to O2 or oxidizing agent is decreased with rise in temperature.

d. EFFECT OF HUMIDITY: - Critical humidity is defined as the relative humidity above which the atmosphere corrosion rate of metal increases sharply. The value of critical humidity depends on nature of metal and corrosion products. Corrosion of a metal is furnished in humid atmosphere because gases (CO2, O2) and vapours present in atmosphere furnish water to the electrolyte essential to establish an electrochemical corrosion cell. The oxide film on the metal surface has the property to absorb moisture. In presence of this absorbed moisture, corrosion rate is enhanced.

CORROSION CONTROL METHODS:

Corrosion is a process very harmful for the metals or alloys. Since the metals and alloys disintegrated or wasted every day through corrosion. It is necessary to protect the metals or alloys against corrosion by some methods. Corrosion protection means not allowing corrosion reaction to take place.

CATHODIC PROTECTION; - the main principle involved in cathodic protection is to force the metal behave like a cathode. Since there will not be any cathodic area without anodic area. Therefore to achieve this auxiliary anode is provided in the corroding medium which is connected to an external source of Current.

There are two types of cathodic protections.

i)sacrificial anode method 	 ii) impressed current method

1.sacrificial anode method

In the sacrificial anode method a more active metal like Zn, Mg, Al, or their alloys is connected to the metal structure to be protected. So that all the corrosion is concentrated at the more active metal and thus saving the metal structure from the corrosion. The electrons in this case are supplied by dissolution of sacrificial anode.  The advantage of this method is it requires low installation and maintenance cost.

The applications are protection of underground cable and pipes from corrosion.

ii) impressed current method

In immersed current method an impressed current is applied to convert the corroding metal from an anode to a cathode. This can be accompanied by applying sufficient amount of direct current from batteries.

LIMITATIONS

A. Capital investment and maintenance cost.

B. It is difficult to maintain uniform current over the entire metal surface and as a result localized corrosion may occur.

C. Chemical reactions occurring at the surface of the protected structure may cause problems. If the cathodic reaction produce hydrogen which may corrode the base

SURFACE COATINGS;-

Another important method for protecting a metal from corrosion is to apply a protective coating. The protective coating may be metallic or non metallic or organic substances.

For applying any type of coating it is very essential to prepare the metal surface properly.

SURFACE PREPARATIONS:

(a) MECHANICAL CLEANING: - this method is useful in removing loose scale and rust. The different methods are hammering, wire brushing, grinding etc.  These are done by instruments like hammer, chisel, brush etc.

(b) SAND BLASTING: - It is a technique of cleaning larger area of surface to produce enough roughness for good adherence of protective coating. In this process sand, along with air steam under a pressure of 25-100 atm is focused on the metal surface.

(c)DEGREASING (SOLVANT CLEANING): - this method is mainly used for removal of oil, grease, rust etc. alcohols, xylene, toluene, acetone etc are used solvent cleaning. This is usually followed by cleaning with hot water or steam.

(d) PICKLING: - in this process the base metal is dipped inside the acid or basic solution for a longer duration. Followed by water cleaning.

METALLIC COATINGS.

Metallic coatings are mostly applied in iron and steel because they are the cheap and commonly used construction materials. There are two methods applied for the application of metallic coatings on base metal.

(1)  Hot dipping    (2) electroplating

(1)  HOT DIPPING: - It is one of the common methods of applying metallic coating on the surface of base metals. In this process the metal to be coated is dipped in the molten bath of the coating metal for sufficient time and then removed out along with the adhering film. Three most widely applied hot dipping methods are galvanizing, tinning and metal cladding.

(a)  GALVANIZING: - zinc coating on a metal surface by hot dipping process is called galvanizing. Zinc coating on iron by hot dipping is the most widely used metallic coating. The metal sheet is cleaned by alkali wash followed by pickling in dilute sulphuric acid and finally washed with water. It is then passed through a bath of molten zinc at about 430ºc. Smaller articles like blots, nuts, etc., are kept in a perforated container and dipped in molten zinc and the excess of zinc is removed by centrifuging. A layer of an alloy of zinc on iron is formed to which the outer layer of zinc sticks the article should not be dipped in zinc at a very high temperature of for a long period as a thick layer of alloy is formed which is brittle. Galvanizing is widely used for protecting iron exposed to the atmosphere.

(b)  TINNING: - Tin coating in a metal surface by hot dipping process is called tinning. In this process the steel or iron sheet is passed through dilute H2SO4 to remove surface impurities. It is then passed through a bath of molten tin which is covered by a flux of ZnCl2. After the coating of tin the sheet is passed through palm oil. The palm oil protects the tin coated hot surface against oxidation. Finally the sheet is passed through rollers to produce the thin coat of tin of uniform thickness. Tinning is widely used for coating the steel copper and brass sheets used for making containers used for storing food stuffs.

(c)  CLADING: - metal cladding is the process of sandwiching the base metal between two thin layers of coating metal by hot rolling at high temperature and pressure. The coat metals are usually metals of least reactivity and higher in the electrochemical series. Ex. Ni, Cu, Ag, Au, Pt etc. the cladding layer should be very thin and its thickness is only 5% of the total composite metal. Duraluminum sandwiched between aluminum sheets and rolled to produce the composite which is free from stress corrosion.

MATERIAL COLLECTED FROM INTERNET/WEBSITES

1.http://chemed.chem.purdue.edu/genchem/topicreview/bp/ch8/mo.html

2. https://youtu.be/FMxuss0RXOU

https://youtu.be/ZFVBR9zW5wU

3. https://youtu.be/Mky1uSLdpSE

4. https://youtu.be/ANr1wgYLVRo

5. https://youtu.be/CWOJW4357Bg

6. https://youtu.be/teTkvUtW4SA

7.https://youtu.be/bG-w6lOHvnM

8. https://www.askiitians.com/iit-jee-chemistry/physical-chemistry/electrochemistry/corrosion/

9. https://www.slideshare.net/DHARMENDRABARIA1/basic-aspects-of-stereochemistry

9. https://people.wou.edu/~courtna/ch334/lecture/lect20.htm

10. https://youtu.be/ywR6aLpfjl0

11. https://www.slideshare.net/LOKESHPANIGRAHI/spectroscopy-134933430

12.https://youtu.be/RuwbeA22rew

POWER POINT PRESENTATION

INNOVATIVE TEACHING METHODS

UNIVERSITY QUESTION PAPERS/QUESTION BANK

- https://firstranker.com/frv2downloads.php/JNTUH/B-Tech/2018/1-1/Dec/R18/151AF-CHEMISTRY-2-R18

2.https://www.manaresults.co.in/jntuh/download1.php?id=152AN052019.pdf&subcode=152AN

3.https://www.manaresults.co.in/jntuh/download1.php?id=152AN082019.pdf&subcode=152AN

REFERENCES

TEXT BOOKS:

Engineering Chemistry by P.C. Jain and M. Jain, Dhanpatrai

Publishing Company, 2010.

Engineering Chemistry by Rama Devi and Rath, Cengage learning 2nd

Edition 2022.

A text book of Engineering Chemistry by M. Thirumala Chary,

E.Laxminarayana and K. Shashikala, Pearson Publications, 2021.

Textbook of Engineering Chemistry by Jaya Shree Anireddy, Wiley

Publications.

REFERENCE BOOKS

TEXT BOOKS:

- Engineering Chemistry by P.C. Jain and M. Jain, Dhanpatrai

Publishing Company, 2010.

- Engineering Chemistry by Rama Devi and Rath, Cengage learning 2nd

Edition 2022.

- A text book of Engineering Chemistry by M. Thirumala Chary,

E.Laxminarayana and K. Shashikala, Pearson Publications, 2021.

- Textbook of Engineering Chemistry by Jaya Shree Anireddy, Wiley

Publications.

REFERENCE BOOKS

- Engineering Chemistry by Shikha Agarwal, Cambridge University

Press, Delhi (2015).

- Text book of Engineering Chemistry by S.S.Dara, S Chand

Publications 2022.

- Polymers: Introduction, classification and mechanism of polymerisation- Addition (Free radical polymerisation mechanism and Condensation polymerisation. Classification of polymers - Thermoplastics & Thermosetting resins. Types of Polymerization of polymers (i) Addition (ii) Condensation (iii) Co-Polymerization.

- Preparation, properties and engineering application of PVC, Teflon and Bakelite. Fibers- characteristics of fibers – preparation, properties and uses of Nylon – 6,6 and Dacron – Fiber Reinforced Plastics (FRP) Glass Fibres – applications. Rubber – Natural rubber and its vulcanization. Synthetic Elastomers- Buna S, Butyl rubber and Thiokol Rubber.

- Conducting polymers: Introduction, classification and mechanism of conduction in Poly-acetylene, applications of conducting polymers.

- Biodegradable polymers: Introduction preparation, properties and applications of polylactic acid